QUICK INSTALLATION GUIDE

IDE/AT

Conner drives are low level formatted at the factory. It is only necessary to run SETUP, FDISK and DOS FORMAT.

Model	Hds	Cyls	 Sec	   Pcomp	L-Zone	Type    Table  LED

CP2034	2	823	 38		0	823	 	*UT		3	n/a

CP2064	4	823	 38		0 	823	 	*UT		3	n/a

CP2084	8	548	 38		0	548	 	*UT		3	n/a

CP2124	8	560	 53		0	560		*UT		3	n/a

CP3184	6 	832	 33		0	832	  	*UT		1	A

CP3104	8	776	 33		0  	776	  	*UT		1	A

CP3204	16	683	 38		0 	683	  	*UT		2	B

CP30064	4	762	 39		0	762	  	*UT		2	B

CP30084	8	526	 39		0	526	  	*UT		2	B

CP30104	8	762	 39		0	762	  	*UT		2	B

CP30084E	4	903	 46		0	903		*UT		3	C

CP30174E	8	903	 46		0	903		*UT		3	C

CP30204	16	683	 38		0	683		*UT		3	C

CP3304	16	659	 63		0	659		*UT		3	D

CP3364	16	702	 63		0	702		*UT		3	D

CP3504	16	987	 63		0	987		*UT		3	D

CP3544	16	1024	 63		0	1024		*UT		3	B

Table 1 jumper settings:

Single drive = Jumper ACT and C/D

Master drive = Jumper ACT, C/D and DSP.

Slave drive = No jumpers installed.

Table 2 jumper settings:

Single drive = Jumper C/D

Master drive = Jumper C/D & DSP

Slave drive = No jumpers installed.

Table 3 jumper settings:

Single and Master drive = Jumper C/D

Slave drive = no jumper

ALL CONNER 20 MBYTE DRIVES USE DRIVE TYPE 2. ALL CONNER 40 MBYTE DRIVES USE DRIVE TYPE 17.								* UT = Universal translate. Select a drive type that is close to, but does not exceed the megabyte capacity of the drive. The drive will translate to the megabyte capacity you have selected.

LED

A: J-4		B: J-1		C: J-5		D: J-3

Pin 1 = +		Pin 3 = +		Pin 3 = +	 	Pin 3 = +

Pin 2 = -		Pin 4 = -		Pin 4 = -		Pin 4 = -

QUICK INSTALLATION GUIDE

SCSI

Most SCSI host adapters are compatible with Conner drives. Software drivers and installation instructions are provided with the host adapter.

The drives are shipped with SCSI ID set to 7. To select a different ID refer to the following:

Table A					Table B

ID	E-1	E-2	E-3			ID	E2	E3	E4

0	out	out	out			0	out	out	out

1	in	out	out			1	in	out	out

2	out	in	out			2	out	in	out

3	in	in	out			3	in	in	out

4	out	out	in			4	out	out	in

5	in	out	in			5	in	out	in

6	out	in	in			6	out	in	in

7	in	in	in			7	in	in	in

Parity is always ENABLED on the CP3200,CP30060,CP30080,CP30100.

All other models, jumper E-4 to disable parity.

SCSI drive parameters:

Model		Hds		Cyl		Sec		Table	LED

CP2020		2		642		32		A		n/a

CP340		4		788		26		B		1

CP3020		2		622		33		A		1

CP3040		2	    1026		40		A		1

CP3180		6		832		33		A		1

CP3100		8		776		33		A		1

CP30060		2	    1524		39		A		2

CP30080		4	    1053		39		A		2

CP30100		4	    1522		39		A		2

CP30200		4	    2119		49		A		2

CP3200		8	    1366		38		A		2

CP3360		8	    1806		49		A		2

CP3540 		12	    1806		49		A		2

LED 1			LED 2

J-4	Pin 1 = +		J-1 Pin 3 = +

Pin 2 = -		    Pin 4 = -

CONTACT YOUR LOCAL DISTRIBUTOR FOR TECHNICAL ASSISTANCE