[Excerpt from an Global AUTOVON Telephone Directory as found in a Fort

Lauderdale library... this information is likely getting dated with

technological advance, but should prove to be interesting history.

Even if all the terms are not self-explained the overall content

should be of interest to telecom enthusiasts.]

AUTOmatic VOice Network (AUTOVON)

I. BRIEF DESCRIPTION OF THE AUTOVON

A. The Global AUTOVON is the principal long-haul, nonsecure, common

user voice communications network for the Department of Defense (DoD).

It provides worldwide direct distance dialing station to station service

through a system of government owned and leased automatic switching and

transmission facilities.

B. At present, the AUTOVON spans the earth from Asia to the Middle East,

and from Alaska to Panama. The AUTOVON has approximately 18,000 subscribers

(direct access to the network). The number of users of the network (those

who must dial an access code or go through an operator to obtain AUTOVON

service) far exceed the number of subscribers. Calls on the network average

about 1.1 million attempts daily with an average call length of 3 to 5

minutes.

C. AUTOVON is a major and integral part of the Defense Communications

System (DCS). It is comprised of all DoD nontactical long-haul

point-to-point communications facilities and personnel. It is the

non-secure common user switched voice network of the DCS.

D. The AUTOVON's primary mission is to provide rapid, world-wide command

and control communications for the National Command Authority (NCA)

and other high priority subscribers. Its secondary mission is to provide

an acceptable grade of service for operational, intelligence, logistic,

administrative, and diplomatic users.

II. NATIONAL COMMUNICATIONS SYSTEM VOICE PRECEDENCE SYSTEM

A. The National Communications System (NCS) voice precedence system,

established by NCS Memorandum 1-70 dated 14 February 1970, is directed

for use by all authorized users of the voice communications facilities

in the DoD. Since the effectiveness of the system depends on the

cooperation of the people authorized to employ it, users must (1) be

familiar with the purpose of each precedence category and the type of

call that is assigned the precedence, and (2) exercise care not to

request or use a precedence higher than required.

B. The NCS voice precedence system does not make provisions for

conducting test and exercise calls. Those activities or individuals

authorized or required to conduct such test or exercise calls will

employ a precedence consistent with the nature of the test or exercise.

When the originator of the test or exercise call has contacted the called

party, the call will immediately be identified as a "FLASH, IMMEDIATE, or

PRIORITY test or exercise."

C. Calls of a given precedence will not normally preempt calls of an

equal precedence. However, calls originated by the President of the

United States, Secretary of Defense and Joint Chiefs of Staff can

preempt FLASH calls in progress by application of their FLASH OVERRIDE

capability. In addition, Commanders of unified and specified commands,

when declaring either Defense Condition One (DEFCON ONE) or Air Defense

Emergency, may preempt FLASH calls in progress by application of the

FLASH OVERRIDE capability.

D. The examples listed below should aid the caller in determining what

precedence to use. These examples are in accordance with the NCS voice

precedence system, but are not to be used exclusively for determination

of the precedence of a call. This should be at the discretion of the

originator of the call.

1. FLASH OVERRIDE  It should be noted that FLASH OVERRIDE is a capability

and is not considered a level of precedence. Exercising this capability

preempts calls in "ALL" levels of precedence. FLASH OVERRIDE calls will be

handled as fast as humanly possible. The FLASH OVERRIDE capacility is

available to the following:

(a) The President of the United States of America

(b) The Secretary  of Defense and Joint Chiefs of Staff.

(c) Commanders of unified and specified commands declaring either

Defense Condition One (DEFCON ONE) or Defense Emergency.

(d) CINCNORAD when declaring either DEFCON ONE or Air Defence Emergency.

2. FLASH  Flash calls preempt IMMEDIATE, PRIORITY, and ROUTINE calls and

will be handled as fast as humanly possible. Listed below are examples

of FLASH calls:

(a) Calls pertaining to command and control of military forces essential

to defense and retaliation.

(b) Critical intelligence essential to National survival.

(c) Conduct of diplomatic negotiations critical to arresting or

limiting hostilities.

(d) Dissemination of critical civil alert information essential to

National survival.

(e) Continuity of Federal Government functions essential to National

survival.

(f) Fulfillment of critical United States internal security functions

essential to National survival.

(g) Catastrophic events of National or International significance.

C. IMMEDIATE  Immediate calls preempt PRIORITY and ROUTINE calls and are

reserved for communications pertaining to situations which GRAVELY affect

the security of Natioanl and Allied forces. These calls will be handled

as fast as possible. Listed below are some exampled of IMMEDIATE calls:

(a) Reconstitution of forces in a post attack period.

(b) Intelligence essential to National Security.

(c) Conduct of diplomatic negotiations to reduce or limit the

threat of war.

(d) Implementation of Federal Government actions essential to

National survival.

(e) Situations which gravely affect the internal security of

the United States.

(f) Civil Defense actions concerning direction of our population

and their survival.

(g) Disaster or events serious enough to have an immediate and

detrimental effect on the welfare of the population.

(h) Vital information having an immediate effect on aircraft,

spacecraft or missile operations.

(i) Distress assistance.

3. PRIORITY   Priority calls preempt ROUTINE calls and are reserved for

communications requiring expeditious action by called parties furnishing

essential information for conducting government operations.

4. ROUTINE  This precedence applies to official Government communications

which require rapid transmission by telephonic means but do not require

preferential handling. A ROUTINE call does not preempt any other call

and is handled sequentially as placed by the calling party.

III.  WHAT CATEGORY OF NETWORK CUSTOMER ARE YOU?

A. You are an AUTOVON SUBSCRIBER if you can make a call on your phone

without going through a local operator ot DO NOT dial a special access

number. This type of telephone is installed for command & control and

other specific operational purposes. More technically, any individual,

post, station or location directly connected to an AUTOVON switching

center is a subscriber. This means a post switchboard connected to the

AUTOVON is also considered a subscriber. Most people who make AUTOVON

telephone calls DO NOT fall into this category. The majority are

AUTOVON users.

B. You are an AUTOVON USER if you dial a special access number before

you can make an AUTOVON call or if an operator must place the AUTOVON

call for you. This distinction between user and subscriber is made

because there are differences in calling capabilities and instructions

found later in this directory.

IV. WHO IS LISTED IN THE GLOBAL AUTOVON DIRECTORY?

A. SUBSCRIBERS are usually the only listing in this directory. If you are

a subscriber and desire to be listed or if you are listed and do not want

to be listed, forward requests to DCA Code B522 Washington, D.C. 20305-2000.

B. USERS are not normally listed in the Global AUTOVON Telephone Directory.

However, the following are exceptions:

1. If a user is served by a switch which has network in/out dial

(NIOD) and is not manned 24 hours a day, calling parties must have a

point of contact when the switch is unattended. Attended nonduty hour

numbers may be listed in addition to the switch number.

2. If a user is an installation which has an individual information

operator number in addition to the assistance operator number, that

information number will be listed.

C. Requests for user listings should be directed in writing to the user's

serving communications facility for consideration by the Telecommunications

Certification Office (TCO). Amplifying information on AUTOVON directory

listings can be found in DCA Circular 310-V55-6.

V. GLOBAL AUTOVON TELEPHONE DIRECTORY DISTRIBUTION

A. It is not practical to publish a Global AUTOVON Directory and provide a

listing of all commands, activities and agencies that have the capability

to access the network. It follows that distributing the directory to all

commands, activities and agencies with the capability to access the system

is costly. Yet, the directory needs the widest possible distribution to be

effective. This problem is recogniized and distribution of the directory is

made to all subscribers either directly or indirectly via major commands and

agencies world wide. All users may purchase the directory from the Government

Printing Office (GPO). If a directory is unavailable, an alternate method to

obtain AUTOVON numbers is to consult your local operator.

B. Publishers of local directories are encouraged to include excerpts

from this directory and locally tailored AUTOVON number lists in their

directories.

C. If you are on the DCA distribution list for this directory and your

address is incorrect, notify DCA, Code B522, Washington, DC, 20305-2000,

using the correction page in this book.

VI. NETWORK RECORDINGS AND SIGNAL TONES

A. Various tones and recorded announcements tell you about the progress

or disposition of your call:

1. When you hear a DIAL TONE, the network is ready for your call. Dial

your number.

2. When you hear a RINGING TONE, your called number is ringing. Wait for

your party to answer. On routine calls, you should hear a normal ringing

tone (10 rings per minute). On calls of higher precedence, you will hear

a very fast ringing tone (30 rings per minute).

3. When you hear a BUSY TONE, your called number is busy. Try again later.

If you hear the busy tone and your called number is not in use then local

or network equipment is busy. Sometimes there is an equipment irregularity.

Try again later.

4. When you hear a PREEMPT TONE, your call was cut off by a higher

precedence call. Hang up. Wait for a moment in case the call is for you.

If the preempted call requires completion, custom dictates that whomever

placed the original call should reestablish it.

5. When you hear a WARBLE TONE, answer your phone and standby for a

conference call or instructions.

USE OF AUTOVON

CONSTITUTES CONSENT TO

COMMUNICATIONS SECURITY MONITORING

B. Below are some of the recorded announcements you might hear while

using the AUTOVON. These announcements should be noted and reported

according to procedures outlined in para XV Telephone Trouble Reporting,

if trouble is suspected.

1. "Your call cannot be completed as dialed. Please consult your directory

and call again or ask your operator for assistance. This is a recording

(pause) SSB number ____."

2. "The precedence used is not authorized for your line. Please use an

authorized precedence or ask your operator for assistance. This is a

recording (pause) SSB number ____."

3. "Equal or higher precedence calls have prevented completion of your

call or the number you have dialled is not equipped for preemption. This

is a recording (pause) SSB number ____."

4. "AUTOVON service disruption has prevented the completion of your

call. Please wait 30 minutes and try again. In case of an emergency,

call your operator. This is a recording (pause) SSB number ____."

VII. CONFERENCE CALLS

A. Dial one of the following numbers and tell the operator you wish to

make a conference call:

CONUS  Alaska  OVERSEAS

1. Subscribers dial     0       0    550-1411

2. Users dial           0       0        0

VIII. CALLS TO COMMERCIAL NUMBERS

Official long-distance calls should be placed using a combination of

AUTOVON and local base switchboard lines where possible. You can use

this procedure if the called base switchboard is permitted by the

commander to connect incoming AUTOVON calls to a commercial number and

the called commercial number is within the toll free radius of the

switchboard. OFF-NET extensions will not be completed unless adequate

call supervision is installed.

IX. CALL ASSISTANCE

AUTOVON Assistance Operators (AAO) are available to assist subscribers

when needed. If you are having trouble completing a call, dial "0" to

reach the AAO.

X. SUBSCRIBER PROVIDED EQUIPMENT

The terminal equipment provided by the subscriber for connection to

an AUTOVON switching center must meet the technical interface criteria

outlined in DCA Circular 310-V175-6, System Interface Criteria.

XI. NEW OR ADDITIONAL SERVICE

In order to obtain a new AUTOVON service or to change the type of service

you now have, consult DCA Circular 310-130-1, Submission of Telecommunications

Service Requests.

XII. GLOBAL AUTOVON CALLING

A. The AUTOVON telephone network provides global telephone service for

the DCS. Some subscribers can call everywhere while others are limited

to one particular area or combination of areas. Where you can call depends

on your mission and whether your telephone line is equipped accordingly.

If your calling area is limited, you will not be able to dial outside of

your limit. The directory indicates calling areas and codes. The area code

need not be dialed for calls within the area. Calls to an area outside

the boundary require an area code plus the seven digit AUTOVON number.

If you are transmitting data, alternate area codes are used. Voice and

Data area codes are listed on the first page of eacg geographical listing.

(NOTE: All CONUS and Alaska subscribers can make respective inter-area

calls by following the above procedures.)

B. Secure calls via an AUTOSEVOCOM telephone will be made in accordance

with instructions in the AUTOSEVOCOM Telephone Directory.

XIII. CALLING PROCEDURES

A. VOICE TELEPHONE CALLS: Listen for the dial tone, depress the appropriate

precedence button if the call is to be higher than routine, and dial the

AUTOVON number as outlined above under global AUTOVON calling.

B. DATA TELEPHONE CALLS: Data telephones and associated data equipment

are operated in accordance with local restrictions. If you send data

via AUTOVON, it is necessary to observe the following restrictions:

1. DO NOT:

(a) transmit more than 18 continuous minutes.

(b) transmit more than one hour during the busy calling time.

(c) use a precedence higher than ROUTINE.

2. DO:

(a) use nonbusy hours for data transmission when possible

(b) dial "11" to properly condition telephone lines for data calls

(c) ensure that the device is equipped with an automatic disconnect

to free the telephone circuit after the device becomes inactive

for one minute.

XV. TELEPHONE TROUBLE REPORTING

A. Ensure that the AUTOVON number you used is correct. If you dialed the

number correctly and you are still having trouble, report the problem as

follows:

1. Keep the line connected if you can. Call the AUTOVON trouble desk at

550-1611 on a different line. (550-1611 will ring at your serving

AUTOVON switch.) Tell the trouble desk attendant:

(a) Your AUTOVON number and location

(b) Called party AUTOVON number and location

(c) What the problem was and when it occurred

(d) Get the attendant's initials then request a call back when

the trouble is repaired.

(e) If you hear an announcement, listen until the end; you will hear

a number (SSB Nr ____). Provide this to the trouble desk. This will

help trace the source of the trouble.

2. If you can not reache the AUTOVON trouble desk, call your local

repair service or the operator using a local base telephone. Be sure to

state that you are reporting an AUTOVON trouble.

REPAIR ITEMS SUCH AS BROKEN PARTS OR FRAYED CORDS ARE NOT AUTOVON PROBLEMS

AND SHOULD BE REFERRED TO THE LOCAL REPAIR SERVICE.

XVI. UNSATISFACTORY SERVICE REPORTING

A. If you are not satisfied with the results when you reported an AUTOVON

trouble, please inform DCSO Code B522 in writing. Keep in mind that we

have to give this report to engineers and technicians for resolution, so

we have to ask for some fairly technical details. If you need help in making

the report, contact you local telecommunications officer or OIC of the

station switchboard. The following information is necessary:

1. Your station name, unit name, AUTOVON telephone number and whether the

trouble concerns a telephone or a switchboard.

2. The time and date you first noticed the problem, the AUTOVON trouble

desk you reported it to and whether the problem was solved.

3. What kind of problem you had and the impact it had on your mission.

B. If you are unhappy with the agency which was supposed to fix the problem,

tell us exactly what unit it is, when you called them, when they arrived

and when/if the problem was repaired. Let us know why you think their

service was unsatisfactory. If this is a repeated problem, inform us

how many times and whether your comments apply to each occurrence.

C. Give us any background on the problem you have including what the

maintenance technicians say the trouble was. Suggest any changes you

would like to see. Pinpoint any trends of performance which may result

in unsatisfactory service.

1. Send the report to the agency nearest you:

(a) Action:

(1) DCA HQ, DCSO

Code B522

Washington, D.C. 20305-2000

(2) DCA EUR

Code E520

APO New York, NY 09131

(3) DCA PAC

Code P420

Wheeler AFB, Hi 96854

(b) Info:

O&M Agency in your area responsible for providing service

XVII. GLOBAL AUTOVON CALLING

USERS are normally limited to placing ROUTINE calls within a geographic

area whether they do the dialling or go through an operator. To call

outside a calling area, dial "0" and ask the switchboard operatore

for assistance.