TN210-7.DOC

USER COMMAND LIST

*********************

TNPlus  version  2.10  has the following commands available  to  the  user:

(Commands may be abbreviated as indicated.)

BYE  (B)  - Use this command when you want the disconnect to  be  initiated

from the far end of the circuit.

CONNECT  (C) - This is the general command to connect to and from the node.

Examples  are:   "C PDT" to connect to a node, "C KB5CDX" to connect  from  the

node to a local station.   Connecting through a digipeating path is disallowed.

CQ  - When the node receives a CQ command, it transmits once:  CQ plus  any

user  text up to a maximum length of 77 characters, including spaces.  The node

is also  conditioned  by this action to display the CQing station in its  USERS

response  for a period of time set by Parameter 24 (usually 15 minutes), unless

the  CQing  station  issues  another command before the time runs out.   Should

another  station  perform  a  USERS  command  during  this  time,  he could see

something like this:

DMN:WD5EZC-2> TheNet Plus 2.10st (673)

Circuit(CLOUD:WD5EZC-1 W2RRY)  <..>  CQ(W2RRY-15)

Anyone  observing  the  CQ  can connect to the node  and  then  connect  to

W2RRY-15  to initiate a QSO.  If the CQing station doesn't get a reply after  a

few minutes,  he can reinitiate the CQ command from time to time  in  hopes  of

attracting  someone on the local frequency.  Remember, each time the CQ command

plus text is given,  it will be broadcast unproto by the node for just that one

time.     If one wants to attract the attention of a monitoring station, the CQ

procedure  can  be  used  to pass a unproto message off a specific node.    For

instance:  "CQ  will  someone  telephone  Pat,  KR5S to turn on his TNC?"   The

request can be repeated every minute or so until the sender is either connected

to, or decides no one is monitoring the local node.

HEARD  (H)  - The "H" command will display level 2 (normal) users heard  by

the node during the past 15 minute period.  Netnodes and level 3 users will not

normally  be  seen  once the  node has "initialized" itself (a 30 -  60  minute

period  after  startup).  The maximum number of users allocated for  the  Heard

table is  20.   Stations listed in  the heard table  are not ranked in order of

time,  i.e., the first station listed may not be either the  most recent or the

oldest  station heard.    If stations  have  not been heard  during the past 15

minutes, a "No One" response is given.

The Heard function compares the  new call with the call signs listed in the

node alias table.   If a match isn't made, the new call will show  on the Heard

list.   Thus there may be brief periods when the call sign of either a new node

or a propagated node may appear on the Heard list.  When a match is made of the

alias, the node call sign will no longer be listed.   This is also true of node

aliases,  which  may  temporarily  show.    Additionally,  the list may include

Ka-node and non-node aliases, if heard.

Callsigns of downlinking stations from local nodes also will be shown.   If

a user  notes  an SSID of -14 or -15  associated with the  callsign,  the  odds

are  these  calls  will be  unconnectable  since  the  originating  station  is

separated by  one or more nodes.   They are picked up in the local node's Heard

list  when  converting to a "Downlink" status in the process of connecting to a

local station.

INFO  (I)  - Is  SYSOP   programmable  and  has  a  capacity  of  up to 160

characters  including control characters and spaces.    The purpose of the INFO

section  is to  provide  local information  and/or news  of interest  to users.

Distant users find it very convenient to know the location of the node and it's

frequency.   Such  data is  useful for  those exploring  and mapping the packet

network.

The INFO section is also a good place to identify a local 24-hour maildrop

or BBS.    Or,  one can place routing information, such as the path to a local

DXCluster or gateway.   In some areas, network management policies dictate the

format and content of node INFO messages.

The procedure for writing to INFO is further explained in the SYSOP COMMAND

LIST.

NODES (N) - This command calls down a listing of the nodes contained in the

destination table.   It gives a user a listing of possible destination nodes to

connect to.  It's good practice to use this command sparingly.     Owing to the

condition  of our congested  networks,  a NODES dump may cause all users on the

circuit  to  be  disconnected  due to the large barrage of packets this command

normally  yields.   Since VHF networks are relatively stable,  chances are, the

contents  of  a local node table are not apt to change much from hour to  hour.

Thus,  local users should be  encouraged to  NOT command a NODES dump every few

minutes.    For the same  reason,  it's a poor practice to request  NODES, from

distant network connects due to the real possibility of disconnecting all users

along a multiple node connection.

Variations  on the N command are:  "N *" and "N <alias or callsign>".   The

"N *"  command  ADDITIONALLY yields a NODES dump of the # (hidden) nodes.   The

hidden  nodes are  those normally  associated with backbone trunking.   A  very

useful  command  is  that  of  "N <alias or callsign>."   This allows a user to

find the path quality and identification  of the nearest  node  leading  to the

destination node requested in the command.  An example:

A  user is  at the  NDGAF  node  and wishes to know the route to NDPMB.  He

issues a "N NDPMB" command and receives this response:

NDGAF:WA0RLE-1> Routes to: NDPMB:WC0M-8

192 5 0 NDPMB

146 5 0 NDPET

This tells him  there is a DIRECT radio path (the 192 and 0) and  that  the

path is fairly recent (as shown by the "5").   He can assume a high probability

of success should he attempt the connect to NDPMB.   He also  notes there  is a

secondary route to NDPMB via NDPET.  The numbers given in the N <alias or call>

command  will  be explained in more detail later.     Here we just want to show

how the N <alias  or  call> command  is a powerful tool,  useful in helping one

navigate throughout the network.