This is the netmail from the Author of BNU.  I have included my BNUPORT.PAT

patch file for your use or examination.

Date: 4:03 am  Thu Mar 4, 1993         Number : 101 of 117

From: David Nugent (3:632/348)         Base   : Private Mail

To  : Don Rieb (115/724)               Refer #: None

Subj: BNU FOSSIL DRIVER                Replies: None

Stat: Private Netmail                  Origin : 01 Mar 93  06:39:53

>          IRQ     IRQ  8259  IO Base  IO Share  Share

>         Mask    Vect Port  Address  Address   Mask

>

> Port 00   10      0C  0020   03F8     0000     00

> Port 01   08      0B  0020   02F8     0000     00

> Port 02   10 *    0D  0020   03E8     0000     00

> Port 03   08 *    71  0020   02E8     0000     00

>

> * What values do I use for the IRQ mask?

Port 02 : Mask 20

03 :      02    8259 Port = 00A0

These are bit masks used to disable/enable the corresponding

interrupt on the 8259A PIC.  Each value is a 1 bit value; that bit

corresponding to the IRQ number:

On the first (and only on XT class machines) 8259A at 0020:

IRQ0=01, IRQ1=02, IRQ2=04, IRQ3=08, IRQ4=10, IRQ5=20, IRQ6=40, IRQ7=80

(Note: for AT class machines IRQ2 cannot be used, use IRQ9 even if

you set the hardware to use IRQ2 - it is redirected to IRQ9)

One the second (on AT class machines and above) at 00A0:

IRQ8=01, IRQ9=02, IRQA=04, IRQB=08, IRQC=10, IRQD=20, IRQE=40, IRQF=80

In general, only IRQ9 is usable on AT class machines if the serial

card is only 8-bits wide or only supports 8-bit operation. I've

never seen a 16-bit serial card, but do understand that they do

exist.

david

______________________________O/______ snip _________________________

O\

Cheers,

david