SimpTerm - A Simple Amateur Radio

Communications Program, Version 2.3

Jim Lynch

February 4, 1996

Contents

1   Introduction                                              2

2   Quick start                                               4

3   Display description                                       5

3.1  Receive window  . . . . . . . . . . . . . . . . . . .5

3.2  Status line . . . . . . . . . . . . . . . . . . . . .5

3.3  Transmit window  . . . . . . . . . . . . . . . . . . 5

4   The Initialization file                                   5

4.1  Keyboard assignments  . . . . . . . . . . . . . . . .5

4.1.1  Special characters and variable substitutions 6

4.1.2  Keyboard macros . . . . . . . . . . . . . . . 8

4.1.3  Assigning files to function keys  . . . . . . 9

4.1.4  Assigning functions to function keys  . . . . 10

4.2  Definition variables  . . . . . . . . . . . . . . . .16

4.3  Color Configuration   . . . . . . . . . . . . . . . .25

4.4  Command line options  . . . . . . . . . . . . . . . .25

5   CD-ROM support                                            27

6   Mouse usage                                               27

7   Pull-down menu                                            28

8   Station logbook                                           28

9   Selective calling feature                                 29

10  License                                                   29

11  In Case of Trouble                                        30

1

12  Error messages                                            31

13  Final notes                                               32

14  Bugs                                                      33

15  Futures                                                   33

1     Introduction

SimpTerm is a simple terminal program designed to be used with

almost any tnc or tu on the market. Features of SimpTerm are:

o Split window operation.

o Macro key definitions.

o User customizable Help screen

o Most of the non-ascii keys can be used as function keys

o Optional scroll back feature on the receive and transmit win-

dows

o Simple status display in the middle of the screen

o Capturing of data to a disk file

o Access DOS commands without dropping communications

connection

o Control of the com port definitions from command line, init

file and keyboard.

o Works on 8088 as well as 80486 and everything in-between.

o Small enough to work well on resource tight platforms, like

laptops.

o Selcal functions, limited unattended operation.

o Times can be in GMT or local time.

o A station logging function.

o Can be set to send CQ or other message repeatedly

o User selectable color scheme

o Automatically senses incoming call letters

2

o Support for call lookup using either QRZ or AmSoft CD-

ROMs

o Pull-down menu access to most functions

o A set of "buttons" used to change modes quickly

o Support for non-standard IRQ and UART I/O base addresses

for COM ports

o Function keys and control keys can be assigned to a macro

string, cause a file to be uploaded or call a function within

the program.

o DesqView aware

There are loads of programs that are tailored to specific tnc's and

there are dozens of good terminal programs that can control any tnc

in a "dumb" (that's a technical term) manner. This program is an

attempt to provide the average ham with something in-between. It

has a lot of features, but not so many as to make use of the program

or the tnc hard. It requires you to know the commands of your tnc,

but also gives you the flexibility that only direct communications

with a tnc can provide.

Statistics

Author:    Jim Lynch

Call:  K4GVO

Address:   128 Carrollwood Dr. Fayetteville, GA 30214

internet address:    k4gvo@america.net (preferred) or jwl@cray.com

If you write, please enclose a sase. If you don't I reserve the right

not to answer.

Requirements

o IBM PC or clone

o DOS 3.1 or greater

o CGA, EGA, VGA or better monitor and graphics card

o 1 Mb disk

o 512K memory

o 8088 or better CPU

o TNC connected to a COM port

3

Development

SimpTerm was developed with:

o Turbo C 2.0

o C/Windows Toolchest, from Mix Software

o EnCom 1.50 COM library, from EnQue Software

The orignal development effort was on a Toshiba T1000SE, and it

runs on my 386sx with a VGA color monitor as well as a 486SLC-

66.

2     Quick start

Edit the simp.ini file and change the line that looks like:

MYCALL NONE

to include your call rather than NONE. Make sure the line that

starts with "COM" looks ok for your setup. If you have any prob-

lems understanding that line, see the discussion on the COM di-

rective later in this document.

Now execute the program. You will see an intro screen followed

shortly by a message instructing you to hit any key.  Do so and

you should see the screen split into two windows with a status line

between them. If you press enter, you will should see the prompt

from your TNC appear in the top window.  If you don't, then a

number of things could be wrong. See the trouble shooting section

of the manual for additional help.

Press the F1 key. You should see a new window appear listing

some of the function keys that are currently assigned to functions

and what those functions do. Press any key to close the help win-

dow.  Press the F3 key.  You should have a screen that displays

small windows of different colors with a number assigned to each.

Also you should see a menu near the bottom center of the screen.

The menu is used to change the colors of the receive, transmit,

status and logging screens.  See the section on configuration for

details.  Press ESC to return the normal display if you like the

default colors.

Press the F10 key. This will bring up a drop-down menu that

will allow you to access many of the functions without remembering

what function keys were assigned. Press the ESC key to close the

menu.

Enter your tnc commands as you would normally do with a

dumb terminal. If you have the echo turned on (in your tnc), you

will see the commands as you type on both screens.

4

Take a look at the simp.ini file and read the section on the

initialization file details. You should be able to customize this file

fairly easily.

3     Display description

There are three displays on the main screen. These are the trans-

mit, receive and status windows.

3.1    Receive window

This is the top window. Data coming back from the RS-232 port

will be displayed here. That is usually the output from the other

station. If you have the echo turned on you will also see the com-

mands that you typed.

3.2    Status line

The screen is divided into two separate windows with a status line

between them.  This status line contains information about the

current configuration of SimpTerm, the status of the selcal features

and other interesting information.

3.3    Transmit window

This is the bottom window.  It displays what you are typing and

some of the error messages reported by SimpTerm.

4     The Initialization file

The initialize file (init file) is used to load the keyboard macros,

assign keys to files,  specify initial com port settings,  specify a

"preload" file for customizing your tnc each time you start SimpTerm

and to define a number of other options. The default init file name

is simp.ini.  This file must be in the directory from which you

started SimpTerm.

4.1    Keyboard assignments

Keys can be assigned to macros, functions or be used to initiate

file uploads.

A full listing of the keys that can be assigned are described in

the keys.doc file found in this distribution. They take the form of:

5

KEY_{modifier_}keyname

The modifier may or may not be necessary, hence the curly

braces, meaning the stuff inside the braces isn't necessary, but

might be important. The modifier is one of the following:

SHIFT    (or SHFT in one case) designates the key must be de-

pressed with the shift key held down.

CTRL     designates the key must be depressed with the control key

held down.

ALT    designates the key must be depressed with the alt key held

down.

A few examples of valid keys are:

o KEY_ALT-X

o KEY_CTRL_P

o KEY_F10

o KEY_SHFT_TAB (this is the only SHFT, the rest are SHIFT)

o KEY_SHIFT_F4

4.1.1   Special characters and variable substitutions

Most of the strings that are defined in the initialization file can

contain special character sequences that permit customization of

those strings and insertion of non-ascii control codes.  A simple

example would be to assign a signature string to a function key

that would be used at the end of each transmission. If you wanted

to send something like:

KD4RQN de K4GVO 12-DEC94 14:22 K

You might have the following in your initialization file.

MYCALL K4GVO

KEY_ALT_F10 ^M%C de %L %D K^M

Anytime you press ALT-F10 you would send the signature line

with the correct date and time.  The following paragraphs detail

the special character sequences.

6

Forming control characters

Control characters are formed by a commonly used notation

where the caret precedes a character to indicate the control code

should be sent and not the character itself.

E. g.

^m   carriage return (enter)

^j  line feed

^d  Control-d

^@   Null (0)

^^   caret

Some of the control characters can be specified via the C lan-

guage backslash escape notation. These are the only ones allowed.

\t  tab

\n  linefeed (new line)

\r  return

\\  backslash

Defining dates and times

Also included are a few time and date functions that can be

specified by a %c notation, where c is one of the following:

D   The date and time in the form dd-MMMyy hh:mm

t  The time in the form hh:mm

d  The date in the form dd-MMMyy

Y  The year in the form yy

M   The month in the form MM

A  The day of the month in the form dd

H   The hour in the form hh

m   The minute in the form mm

S  The second in the form ss

Where:

7

dd  is the day of the month,

MMM      is the 3 Character abbreviation of the month

yy  is the 2 digit year

hh  is the hour

mm    is the minute

MM    is the month (1-12)

Other substitutions

If you have used the mouse or the call menu to select an active

call that is currently being displayed on the status line (column 2),

then you can use that call in a string with the %C substitution.

See the example at the begining of this section.

A %L will be replaced with the value of the MYCALL variable.

See the example at the begining of this section.

4.1.2   Keyboard macros

Keyboard macros are strings that can be assigned to many of the

keys on your keyboard. This lets you assign frequently used char-

acter strings to a single key to save keystrokes. For instance, you

might want to assign your call to the F1 key and your name to the

F2 key and your QTH to the F3 key.  That way when you were

talking to someone (with your fingers, that is) you would just type

"Hi, this is <F1>, <F2> in <F3>" and your computer would fill

in the right stuff. <F1> in the above example means that you hit

the F1 key, not that you typed the string "<F1>". Bear with me

you old timers, I want to make sure the newcomers don't give up

on us.

To define a macro you need to place the key name (yes, it must

be in upper case) in column one, followed by a space, followed by

the macro replacement string.

A simple example based on the earlier discussion would be:

o KEY_F1 K4GVO

o KEY_F2 Jim

o KEY_F3 Fayetteville, GA

Now when you press <F1> <F2> <F3>

K4GVO JIM Fayetteville, GA

8

will get sent to the tnc.  I hope you were connected to someone,

'cause sending that garbage to the tnc in command mode won't be

very productive.

The maximum length of a string that can be assigned to a key

is 511 characters.  You can continue the string on the next line

by placing a backslash (\) character at the very end of a line and

continuing with the definition on the next line.  Be careful not to

add a space after the \ because you won't be able to see it and the

program will think the next line is a new macro definition.  Note

that there will be no line breaks sent at the end of the line.

Example:

KEY_ALT_C cq cq cq cq cq cq de k4gvo \

cq cq cq cq cq cq de k4gvo \

cq cq cq de k4gvo k

This is an example of a complete definition. Note that the first

two lines have the backslash but the last one doesn't. This would

be ok for a cw call, but you probably want to be able to insert a

new line occasionally for rtty, amtor or packet work.

You can insert any control character, including a carriage return

(that is what you normally send when you hit the enter key). Let's

try the above example with the addition of a carriage return.

Example:

KEY_ALT_C cq cq cq cq cq cq de k4gvo^M\

cq cq cq cq cq cq de k4gvo^M\

cq cq cq de k4gvo k^M

Now we have a reasonably constructed macro that won't offend the

station on the other end.

A control character is formed by preceding a letter with a caret

as shown above.  The control characters ctrl-a through ctrl-z are

formed as ^A through ^Z. If you want to include the caret as a

character to be send, use a double caret (^^).  Since there is no

such thing as a control-caret, the program will know to send a single

caret.  You can also specify other special characters and strings.

See the section on selective call reply strings for an expanded set

of options within the macro string.

4.1.3   Assigning files to function keys

A function key can be assigned to a file by preceding the key name

with a ampersand (@). The name of the file follows the key name.

Hence:

9

@KEY_ALT_U mydefs.txt

@KEY_HOME c:\bob\mydefs.txt

are valid definitions. When the specified key is depressed, the file

is uploaded to the tnc. This means that the contents of the file are

sent to the tnc.  This is handy for defining a set of parameters to

customize your tnc for a specific mode of operation. You could use

the macros to do this, but they are limited to only 511 characters.

Another use for this is the "brag tape" (the word tape left over from

when we used paper tape on the old *eletype machines). You could

put a description of your station, location, personal information on

a file and by hitting a single key, you can send it to the other

station.

4.1.4   Assigning functions to function keys

That sounds like a redundant statement doesn't it? It's not really.

There are over thiry functions available that can be assigned to

function keys. These are:

auto        start an automatic send loop

baudup      increment the communications baud rate

bauddown    decrement the communications baud rate

confcom     bring up a menu for com parameter selection

config      configure the colors of the various parts of the

screen

dispbut     display a user configurable menu (See BUTTON)

dos         jump to dos

getcall     display a menu of the calls that have been heard

help        display the help screen

keybrcl     reload a input buffer

logit       brings up a data entry form for the station log

macro       assign a macro to a key on the fly

menu        display a pull-down menu of all of the above

functions

quit        exit the program. Not a bad function to have.

reset       reset the communications port

rforward    scroll the receive window forward one page

rforward1   scroll the receive window forward one line

rback       scroll the receive window back one page

rback1      scroll the receive window back one line

10

rlarger     make the receive window one line larger

rsmaller    make the receive window one line smaller

savebuf     save the receive buffer in a file

status      print status of selcal

tkeybuf     toggle storing a input buffer

togsel      toggle looking for selcal strings

togcap      toggle logging of the incoming data.

upload      upload a file, prompts for name

xback       scroll the transmit window back one page

xback1      scroll the transmit window back one line

xforward    scroll the transmit window forward one page

xforward1   scroll the transmit window forward one line

The logit function displays the station logging screen. Refer to

the station logbook section of the manual for details.

The togcap function will either ask for the name of a capture

file to start saving data to or close that file if capturing had already

been started by a previous call to togcap. The default capture file

name is simp.cap. You may override that name by typing in a new

name over top of the simp.cap entry. While you are capturing, the

phrase "Log on" is displayed on the status line. When you are not

capturing, the status lines displays "Log off". Capture is done in

text mode which does not pass special characters transparently. If

you wish to capture the data in transparent mode you should put

a BINARYCAP statement in your .ini file.

The help function is a rather simple one. It just opens a window

on the screen and dumps the contents of the help file into the

window.  If the help file is too long, the top part of the screen is

scrolled off into never never land. Using a text editor of your choice

you may edit this file as you see fit. The default help file is named

simp.hlp, but you can tell SimpTerm to use another file via the

HELP customization option in the init file. See the init file section

for details.  Also the full text of this document is available under

the Help pull-down menu.

The getcall brings up a menu that lets you select a call from a

list of recently heard calls. After selection the other call will be dis-

played on the status line. If you send a macro string that contains

the special character string, %C, this call will be substituted for

it.  You also can select a call moving the mouse cursor to the first

letter of the call and pressing and releasing the left mouse button.

To clear the status line, just move the mouse to an empty part of

11

the screen and depress the left button. You don't have to clear the

field before selecting a new call however.

The lookup function will attempt to find an entry matching

the call that is displayed at column 2 of the status bar in a CD-

ROM database. The call displayed on the status bar was either as

a result of clicking on the first letter of the call with the left mouse

button or from invoking the getcall function.  Use of this function

requires that either the QRZ or AMSOFT parameter be used to

specify the path to the CD-ROM drive and the CD present.

The dos function brings up a second copy of the command

processor.  This allows you to execute commands just as if you

had exited the program, but the program is still in memory and

waiting for you to finish. After you have finished with dos and want

to return to SimpTerm, just enter "exit". That will bring you back

to SimpTerm where you left off.  DON'T type simp again!  The

communications buffer can hold 2048 characters.  That is about

one screenful.  If you stay away too long, you may lose data.  Be

careful.

There are 8 scrolling functions. These are:

rforward, rforward1, rback, rback1, xforward, xforward1,

xback xback1

The rforward and xforward functions cause the receive or

transmit window to scroll forward one page. The rforward1 and

xforward1 functions cause the receive or transmit screens to scroll

forward one line.

The rback, rback1, xback and xback1 functions perform the

reverse operations respectively.

The baudup function will increase the baud rate one step. It

will cycle through the following baud rates, 300, 1200 2400, 4800

and 9600 in a circular manner.  If you must use other rates, you

will have to set them via the COM input line in the init file or use

the confcom function.

The bauddown function will decrease the baud rate one step.

It will cycle through the same baud rates as described in the pre-

vious paragraph.

The rsmaller function will reduce the size of the receive window

by one line. This has the effect of moving the status line up one line

and increasing the size of the transmit window by one line.  The

rlarger function has the reverse effect. You cannot reduce either

window below 3 lines high.

The menu function is used to bring up the pull-down menu.

See the section on the pull-down menu for more information.

The macro function is used to define a macro and assign it to

12

a key without leaving the program. You are first asked to press a

key. This is the key to which the macro will be defined. I suggest

you don't use a printable key, like a letter or a number, but that is

a legal key. You can use this to assign macros to keys that aren't

on the list of valid keys as long as the program recognizes that a

key has been pressed.

If the selected key already has a string assigned to it, you will

be able to edit that string with some limitations. The edit screen

is not a fully functional editor.  It is still under development.  If

the string is longer than 77 characters it will be truncated to 77

characters and they will be dislayed on the first line of the edit

screen.

You will not be able to position the cursor in the middle of a

string and press enter to insert a return in the string. The cursor

will simply move to the next line. All of the lines of the edit window

will be concatenated into a single string, so if you want carriage

returns to be sent to the tnc, remember to use one of the special

characters, ^m or \r.  When you are finished editing the string,

press the ESC key.  The string will be saved.  There is no way to

cancel the operation and return the key to it's original function

after you have selected a key, but if you press enter when it asks

for the key, it won't bring up the edit window.

The auto function starts an automatic transmit loop.  It was

intended to provide a way for the user to send cq periodically but

could be used for other things. Here is how to implement it. First

assign the auto function to a key,

!KEY_ALT_A auto

Then assign a macro to another key with the message/cq/call

that you want repeated periodically.

KEY_ALT_C ^Mx^M\

cq cq cq cq cq cq de k4gvo^M\

cq cq cq cq cq cq de k4gvo^M\

cq cq cq de k4gvo at %t k^M^M^D

Now include a loop time via

AUTOTIME 120

When you run the program and press the ALT-A key, which

invokes the auto function, you will be prompted to enter the key

that you want executed. If, in this case, you want to send cq, you

would merely press the ALT-C key.  If you decided that the 120

seconds was too long to wait, you could press the ENTER key and

13

you would be prompted for a different time delay. If you want to

abort the auto function press the space bar. Let's say you pressed

the ALT-C key.  Your transmitter will come on, send the cq and

turn off.  120 seconds after it started transmitting, it will do the

same thing. And 120 seconds later and ...

Remember that the time is from the beginning of the trans-

mission to the beginning of the next transmission.  If the time is

shorter than the time to transmit, there won't be any delay and

the program will transmit forever, unless you stop it.

If you get tired of calling, or receive a reply, you can stop the

loop by pressing any key.  If the program is transmitting, it will

continue until it is done and then terminate.

While you are in automatic operation you will see a capital

letter A in colum 2 of the status line to remind you.

In the above example above I showed a working sample for a

PK-232.  On the PK-232 the letter X turns the transmitter on in

RTTY mode and the control-D (^D) turns it off after emptying

the buffer. If you are using another model of TNC, you may have

to substitute other character sequences for these.

Some of the TNCs have a "preload transmit buffer" function

that allows you to type-ahead while the other station is sending.

Unfortunately, when you send a character to the TNC it blocks the

incoming characters until you press a ENTER key. If you are a slow

typist, this might cause multiple lines to back up and you might

not see that he has turned it over to you.  A feature that buffers

up your key strokes in the program can be used to alleviate that

situtation.  The tkeybuf function toggles saving key strokes in a

buffer until a specific key is pressed, normally the ENTER key, but

it can be assigned to most any function or control key. When you

hit the specific key, which is assigned via the SENDKEY variable

in the init file, the saved keystrokes are sent to the TNC. You can

save up to 2047 key strokes before you send them.  That is more

than a full 80 by 25 screen full, so should be sufficient for most

applications.  When SimpTerm is buffering keystrokes, a letter K

will be displayed in column 2 of the status line.

If the SENDKEY key is the ENTER key, then it is passed along

with the buffer, but if it is something else, it is not sent, so don't

assign a key to SENDKEY that you want to be able to send.

If you want to reload the buffer with the data you just typed

in and then sent to the TNC only to discover that you were in

command mode rather than type-ahead, you can press the key

associated with the keybrcl function.  Since you also probably

want to put the TNC in type-ahead mode, you will want to hit

the key associated with the tkeybuf function first.  Then enter

the type-ahead command. Then put it back in buffered mode with

another tkeybuf function key and then use the keybrcl function.

14

I know it sound complicated, but it sure is better than retyping all

that stuff you just lost.

For example if the SENDKEY variable looks like:

SENDKEY KEY_ENTER

You can type what you want without interrupting the output

from the TNC until you press the ENTER key. At that time all of

the keystrokes are sent to the TNC.

The upload function lets you upload any file to the tnc. You

will be prompted for the file name.  Normally the upload will be

done in text mode, which eliminates some control characters.  If

you want to upload files in binary or transparent mode, include a

BINARYLOAD statement in the .ini file.

The confcom function brings up a menu screen with radio

buttons to use to configure the com port.  You may find it isn't

worth much unless you have a mouse.

To assign a function key to one of these functions, precede the

definition by an exclamation point (!).

E. g.

!KEY_ALT_X quit

!KEY_CTRL_L togcap

!KEY_ALT_H help

!KEY_F10 togsel

!KEY_ALT_S status

!KEY_ALT_J dos (Jump to dos)

The simp.ini file included in the distribution assigns the page-

down and pageup keys to the rforward and rback functions respec-

tively. Also the up arrow and down arrow keys are assigned to the

rback1 and rforward1 functions.

Just in case you don't have the quit function assigned to a key,

the END key has been permanently bound to the quit function.

So you don't accidently hit it and drop out of the program, the

end key must be pressed twice in succession to end the program.

You can't assign the KEY_END key to anything else. The program

won't complain if you try, but it just won't work the way you think

it should.

There are a few other keys that are special.

KEY_BKSP and KEY_CTRL_H

are intra-line edit functions and cannot be reassigned.

15

KEY_ENTER

can't be reassigned either, obviously.

4.2    Definition variables

You can set a number of configuration options by defining variables

in the initialization file. This section discusses them.

Comment lines

If the program finds a # (pound) sign in the first column of a

line in the init file, it will stop processing that line.  This is the

way to comment your init file.  Also any of the options described

below that have a fixed format will allow comments following all

of the permitted arguments. If an option has a variable number of

arguments, you can't put a comment on that line.

E. g.

# this is a valid comment

mid 15 # this is ok also.

COM port definition option

The COM port parameters must be set to match your tnc.  If

you don't know what speed, word size, etc. that your tnc is using,

consult the manual that came with the tnc.  A lot of the packet

tnc's around have an autobaud detect feature.  When you power

them up for the first time they wait until you type an asterisk and

then they can determine the operating parameters you are using.

If you have both a COM option line in your init file and a

command line com option then the command line option will be

used. This way the command line can be used to override the init

file if desired.

If you don't tell SimpTerm what parameters to use it will default

to:

use COM1

2400 bits per second (baud)

8 data bits

No parity

1 stop bit.

If you include the line,

COM1 4800,7,E,1

it will change the COM1 port to work with:

16

4800 bits per second (baud)

7 data bits

even parity

1 stop bit.

See the section on command line options for the full set of op-

tions for each of these parameter fields. There must be at least 4

fields.  The COM? line in the initial file requires a SPACE imme-

diately after the COM? field.  The command line version of this

command uses a COMMA between these fields.  Try not to get

these confused. Of course, if you have your tnc connected to one of

the other com ports(like 3), you will want to use a line that looks

like:

COM3 9600,8,N,1

instead.

Examples:

COM4 1200,7,N,1,5,0x3E8

COM1 19200,8,N,1

FLOW control specification

There are three options for flow control. You may use RTS/CTS,

XON/XOFF or no flow control. Specify:

FLOW RTS

FLOW XOFF

FLOW NONE

to select one of the three. Default is FLOW RTS.

HELP file specification

The default HELP file name may be overridden by including

the HELP configuration line in the init file. This is simply the key

word HELP followed by a file name.  When you invoke the help

function, the contents of this file are displayed on the screen. You

may modify this file for your own purposes. See the section on the

help function. The default name for the help file is simp.hlp.

17

MID screen layout

One of the feature of SimpTerm is that is has a split screen. This

allows you to have a "transmit" window that displays what you

are typing along with informative messages from the program and

a "receive" window that displays the data coming back from your

tnc. The size of the windows is determined by the mid command

line option. This value should be between 4 and 20. A value of 15

will give you a 15 line transmit window and a 9 line receive window.

If you specify a value for MID out side of this range a default will

be chosen for you.

DEL_EQ_BKSP specification

If you include this definition in your initial file the delete key

will work just like the backspace key. If you are used to using the

delete key rather than the backspace key, you will want use this

keyword.

NODV disable DesqView sense

The inclusion of the NODV line in the .ini file will tell SimpTerm

to act as if DesqView was not loaded.  Normally SimpTerm will

check for the presence of DesqView and yield to the multi-tasking

kernel periodically. If you include NODV SimpTerm will not yield.

Use of this parameter is not recommended for normal operation.

PRELOAD file specification

There is an option available that lets you send a file to the

tnc before control is turned over to you at the keyboard.  The

PRELOAD option line specifies the name of this file.  It follows

the same form as the HELP file spec.  This option may be used

to force certain operating parameters when starting up SimpTerm.

One use of this feature would be to keep different init files each

with a different PRELOAD file.  One PRELOAD file might set

the tnc to work hf, while another might specify vhf.  If you have

two hams that use the same equipment, you might want each to

have his/her own custom PRELOAD file to change, at least, the

call letters.  The PRELOAD file may contain the special format

characters so you could have a line that looked like:

DAYTIME %Y%M%A%H%m

and it would set the date and time on your TNC, if it conformed

to the TNC-2 standard.

18

POSTLOAD file specification

This option names a file that will be uploaded to the tnc at exit

time. If you use this option to reset some parameters on your TNC,

you should precede the parameters with your escape character to

be sure that you are at a command prompt. The POSTLOAD file

is scanned for the special format characters.

NOSCROLLBAR specification

Use this parameter in your .ini file to turn off the scroll bar.

This will give you another column on your screen.

SCROLLLINES count specification

This can be set to a number not to exceed 409.  It is used to

specify the number of scrolllines in each of the window buffers.  I

don't know what would happen if you tried to set them to a small

number, like 4. The default is 300. Bump it up if you want to scroll

back farther and reduce it if you need to recover some memory.

CDSWAP specification

Use this in your .ini file to force SimpTerm to make more mem-

ory available for the CD-ROM database access by swapping. It will

cause the access to run slower than normal so don't use it unless

you are getting an indication that you have run out of memory

while accessing a CD-ROM.

DEBUG specification

To cause the last key character code to be displayed at the end

of the status line, include:

DEBUG KEYS

in your initial file.

TIMELINE display specification

You can tell SimpTerm to display the date and time on the

status line by including this option in your initial file. This line is

searched for the special format characters before being displayed,

so a line that looks like:

TIMELINE %A-%M-%Y %H:%m:%S

in the initial file will cause the status line to display:

25-12-93 12:05:00

19

At 5 minutes past noon on Christmas, 1993, if you're lucky

enough to be able to get on the air on Christmas.

AUTOTIME time specification

AUTOTIME is used in conjunction with the auto function to

implement a semi-automated station operation, like a "cq" loop.

The AUTOTIME line specifies the number of seconds between loop

starts. See the section on the auto function for details.

SELFILE file specification

This line specifies the name of the file that will receive the

selcal messages. The default is "simp.sel". If you want to specify

a different file name and/or drive specification put the SELFILE

line in the init file.

E. g.

SELFILE a:\messages

The messages that are stored in this file are appended to any

previous messages, so you will not lose any previously recorded

messages but you might want to look at the size of this file occa-

sionally so you don't run yourself out of disk space.

GMT time specification line

If the program finds this line in the init file, it will attempt to

display the times in GMT. Since the time function is only used with

selective calling functions, non-selcal users can ignore this option.

In order to make these functions work right, you must set your dos

clock to your local time and set the environment variable TZ to

indicate your local time zone relative to GMT. The format of the

TZ variable is:

TZ=zzz[+/-]d[d][lll]

Where:

zzz  is a 3 character string representing the name of the current

time zone. E. g. CST or EST.

[+/-  d[d]] is a optionally signed number with 1 or 2 digits.  This

number is the local time zone's difference from GMT in hours.

Positive numbers adjust westward from GMT

lll is an optional 3 character field that represents the local time

zone daylight savings time. E. g. PDT Environment variables

are either set in your autoexec.bat file, or can manually be

typed in before you execute the program. Consult your DOS

manual for more information.

20

Examples (DOS command line):

SET TZ=EST5EDT           (New York)

SET TZ=CST6        (Indiana, they don't used daylight time, do they?)

SET TZ=PST8PDT           (Los Angeles)

Because of the "new" rules surrounding daylight time,  you

might find that the time is an hour off during some parts of the

year.  You might want to keep your dos clock in GMT, or adjust

your TZ environment to give you the correct GMT value.

SELREPLY reply line

When a remote station sends you a specific select, i. e. a request

for you to start recording an incoming message, it will expect you to

reply immediately with a message so it doesn't waste time sending

you a message if your equipment isn't monitoring.  This option

specifies the reply that you will give in response to the select.

The format of this reply line can contain ascii characters, control

characters, or special formatting characters.  The ascii characters

are the normal printable characters (or subset in the case of baudot

or amtor).

Example:

SELREPLY x\r^O^O^O^O^O^Ode K4GVO, %D\r^D

Using the PK-232, you can send the "letters" characters with

the control-O character, so to make sure that the other station

has gotten his transmitter shutoff, you might want to delay your

transmission a bit via this method.  The "x" character, followed

by the carriage return tells the PK-232 to start transmitting. The

ctrl-D at the end tells the PK-232 to stop transmitting and resume

receiving.

RSELCAL selective call string option

This line specifies the selcal string(s) that you want to react

to. You may specify up to 5 strings. The strings can be delimited

by spaces, commas or tabs.  The strings can NOT contain any of

these characters. Since these strings should probably contain your

call or your call and some trailing characters, this shouldn't be a

problem.  These strings ARE case sensitive.  That means that if

you are running in a mode that supports upper and lower case,

you had better put your selcal in both upper and lower case and

hope that the sending station doesn't send mixed case. All strings

21

identified from this set of strings by the program will be replied to

with the SELREPLY string.

Example:

RSELCAL k4gvo K4GVO k4gvo-9 kd4rqn

ALLCAL selcal option

This option specifies the set of strings that the program will

look for to start a "all call" recording. When the program finds a

string in this group, it starts recording a message without sending

a reply.  Other than that, it behaves in exactly the same manner

as the RSELCAL option.

Example:

ALLCAL ANYONELISTENING TOALLOFYOU allcal

ENDCALL selcal option

Obviously there must be a way to tell the program to stop

recording a message and one of these ways is to send a stop string.

This option specifies a group of strings that will be used to stop

the recording of the incoming message. This the format for these

strings is the same as the previous two.

Example:

ENDCAL NNNN nnnn ZCZC zczc

SELMAXT selcal time limit option

So that the program doesn't record reams of garbage, a time

limit can be specified. This time limit is in seconds and will only

record for "n" seconds. If it hasn't received a ENDCAL string by

then, it will close the received file. A line will be placed in the file

indicating that the recording was terminated because of a timeout.

Examples:

SELMAXT 300 (record for 5 minutes, maximum)

SELMAXT 32000 (record for virtually forever)

SELMAXT 12 (record only very short messages!)

EXTEND time limit extension string

Now it is nice to limit your messages to a specific time limit,

but if someone just has to get a long message to you, he(she) will

have to send it in chunks. He(she) might not know what you have

set your time limit to, so he might have to send it in a lot of little

22

chunks to be sure that he doesn't time out. In order to avoid that

problem, the EXTEND option can specify a string that the remote

station can send to you to tell you to set your time limit to a

different value. The format of the EXTEND option is:

EXTEND <string> <newtimelimit>

Where:

string  is a group of characters (not including space and tab)

newtimelimit     is the absolute maximum that you want someone

to set your timelimit to.

An example is in order. If you have the following in your init file,

EXTEND NEWTIMEX 600

and a remote station sends your selcal string and then (after wait-

ing for your reply) sends a line that looks like: NEWTIMEX 300^m

(remember the ^m is an enter or end-of-line character) your pro-

gram will set your time limit to 300, giving the remote station the

opportunity to send you up to 5 minutes worth of data. If he(she)

had specified EXTEND NEWTX 4000, the program would limit

the time to 10 minutes, based on the absolute limit you placed on

the time of 600 seconds.

Of course, the other station should be aware of your extend

string. If this string comes in while you are not recording a message,

it will be ignored.

FLATLOG

Use this parameter by itself to indicate that you want the sta-

tion log to be kept in a "flat" file format. In this case, flat means

that the data is in fixed columns. This might be useful for someone

that wanted to process the log from a program and didn't want to

deal with the "comma delimited" format that is the default.

SNOW

Some of the older CGA cards would produce snow on the screen

when direct video writes were done. If you see snow on your screen,

try to insert the snow directive in your simp.ini file.

BIOS

If you have a "near compatible" PC and the display isn't func-

tioning correctly, try to use the bios method of writing data to

the screen. It is fairly slow, but if that is all you have, it's better

than nothing. Insert the bios directive in your simp.ini if you are

still having trouble with snow. Don't have both active at the same

23

time. Include either snow or bios; or don't put anything. WARN-

ING: if you use the bios option, ANSI graphics won't be displayed

properly.

NOMOUSE

If you have a mouse, you will see a scrollbar to the right of the

receiving (top) screen. If you find that having a mouse causes you

problems, insert the nomouse directive to cause the program to

treat you as if you didn't have a mouse.

QUIET

If you don't like the annoying startup screen telling you what

you are running and who's sweat gave it to you, insert the quiet

directive in your init file.

DDMMYY

Just in case someone wants to use this program in Europe

or wherever else they use the day-month-year format, instead of

the month-day-year used here, they can put this directive in their

startup file and the program will generate the date October 31,

1993, as 31/10/93.

YYMMDD

This format is also included for completeness.  It is left to the

reader to determine what it means. (Hint: read the previous para-

graph)

LOGFILE

This directive specifies the name of the station log. The default

is station.log. Example:

LOGFILE k4gvo.log

BUTTON

This directive is used to define one of a set of buttons that can

be used to send commands to your tnc.  Multiple BUTTON vari-

ables may be defined and when the dispbut function is executed,

a window will be displayed with a button for representing each of

the BUTTON variables encountered in the initial file. The format

of the BUTTON variable line is:

BUTTON <label> <string>

24

Where <label> is the 1-8 character label assigned to button

and the <string> is the character string that will be sent to the

tnc when the button is pressed.

Here is a sample set of BUTTON variables.

BUTTON Amtor ^camtor^m

BUTTON Cw ^cmorse^mmspeed 20^m

BUTTON Pac_vhf ^cpacket^mvhf on^mhbaud 1200^m

BUTTON Pac_hf ^cpacket^mvhf off^mhbaud 300^m

BUTTON Amtor ^camtor^m

When you execute the dispbut function, a window with 5 but-

tons will be displayed. Select one or press the escape key to close

the window without executing anything.

4.3    Color Configuration

If you want to change the color scheme used by SimpTerm, you

may use the config function. A number of foreground/background

color combinations will be presented on the screen with a number

identifying each one.  A menu will also appear with a number of

options. Select one of the window options to change the colors for

that window by moving to the option with your up/down arrow

keys and press return. You will be prompted for a number repre-

senting the color you want to use. The other options on the menu

let you save the color scheme that you have selected permanently

or discard the changes or just use the new colors during the current

session.  Permanent screen colors are saved in the simp.cfg file.

Distributed with SimpTerm is a simp.cfg suitable for a color mon-

itor and a mono version, simpmono.cfg.  If you want to use the

mono version, just copy simpmono.cfg to simp.cfg.

4.4    Command line options

The SimpTerm command line looks like:

simp -init initfile -mid number -com comparamstring -word -V -

quiet -cap cfile

Where:

quiet  tells the program not to display the introductory screen. See

the QUIET directive for more details.

word   causes displayed data in a window to wrap on a word boundry.

V  display the version number.

cfile  start the program with capture on and write to cfile

25

initfile  is the name of a file to be used as the initialization file

containing the key definitions and other customization pa-

rameters. The default file name is "simp.ini".

number    is the line number of the middle of the screen.  This is

used to define the location of "split" between the transmit

and receive windows. Default is 12. See the MID discussion

elsewhere.

comparamstring      is the communications port parameter string.

This specification overrides the COM init file option.

The comparamstring parameter is in the form:

comn,baud,wordsize,parity,stopbits,[base],[irq]

Where:

comn   is the com port number, 1, 2, 3 or 4. This also can be

specified as COM1, COM2, COM3 or COM4.

baud   is the baud rate. Must be one of, 300, 1200, 2400, 4800

or 9600.

wordsize   is either 7 or 8.

parity  is one of the following: N, O, E, M, S.

stopbits  is either 1 or 2. This is almost always 1.

base  is the base address of the COM port.  This can be

expressed as a c language hex number such as 0x03e8 or

a decimal number like 123.

irq is the interrupt number used by the COM port. This can

be expressed as a c language hex number such as 0x0a

or a decimal number like 13.

The following settings are the most common. If you don't know

for sure what to set them to, try these.

<comn>,<baud>,7,E,1

<comn>,<baud>,8,N,1

and maybe

<comn>,<baud>,7,O,1

The <comn> and <baud> depend on your computer hardware

configuration and your tnc interface speed.

You may use spaces instead of commas to separate the COM

fields. If you do, you must enclose the entire field in quotes, such

as:

simp -com "COM1 4800 8 N 1"

If you have both a COM option line in your init file and a command

line com option then the command line option will be used. This

way the command line can be used to override the init file if desired.

26

5     CD-ROM support

This version of SimpTerm supports a lookup function that can be

used to find calls in the FCC database. Both the QRZ and AmSoft

CD-ROMs are supported.  To use this feature you will need to

include a line in your .ini file that looks like:

QRZ d:

or

AMSOFT d:

Where d: is the drive and optionally path to the database. For the

July 1995 QRZ CD on CD-ROM drive G: you would use:

QRZ G:

For the September 1995 AmSoft CD on CD-ROM drive G: you

would use:

AMSOFT G:

The lookup function will attempt to find an entry matching

the call that is displayed at column 2 of the status bar in a CD-

ROM database. The call displayed on the status bar was either as

a result of clicking on the first letter of the call with the left mouse

button or from invoking the getcall function.  Use of this function

requires that either the QRZ or AMSOFT parameter be used to

specify the path to the CD-ROM drive and the CD present.

6     Mouse usage

Most of this information is covered elsewhere, so if you need more

information, check the index.

The mouse can be disabled with the NOMOUSE parameter in

the .ini file. Use this feature if SimpTerm erroneously senses that a

mouse is attached or use of the mouse interfers with the operation.

Clicking on a call in the receive window will select that call and

place it near the left edge of the status bar. Operations using the

active call such as invoking the lookup function or sending a string

with the use this selected call.

If you are using scroll bars on the windows, moving the mouse

cursor to either the arrow at the top of the window or the bottom

of the window and clicking the left button will cause the screen

to scroll one line up or down.  If the left button is clicked in the

region below the thumb or slide bar the window is scrolled down a

page and if the button is clicked in the region above the thumb the

window is scrolled up one page.  By moving the mouse cursor to

27

the thumb and pressing and holding the mouse button, the thumb

can be dragged up or down, scrolling the window as it is dragged.

Note that when the screen is displaying scrolled data, incoming

data is not displayed.  To see the incoming data be sure to scroll

back down to the bottom or most recently displayed page.

The pull-down menu can be displayed by pressing the middle

button on a 3 button mouse.  Many 2 button mice can emulate a

middle button by pressing and releasing both buttons at the same

time. Be sure to close the menu with the escape key when you are

finished.

7     Pull-down menu

The pull-down menu is accessed via the menu function. To use it

you must have menu assigned to a function key. The middle button

on a 3 button mouse will also bring up the pull-down menu. If you

have used pull-down or drop-down menus before, you shouldn't

have any trouble using this one. While the menu is being displayed

you will see incoming data from the serial port, but you can't enter

anything into the transmit buffer.  To close the menu, press the

Escape key. This also is used to close a sub-menu and return you

to the drop-down menu.  If you have both the main menu and a

sub-menu displayed, you must press the Escape key twice to return

to normal operation.  You may also use the mouse to select from

the menu items.

8     Station logbook

The station logging screen allows you to enter a number of possibly

helpful items.  The first two fields will be filled in for you as they

are the current date and time.  The cursor will be positioned at

the third field, but if you want to change the previous fields, just

press the CTRL-TAB key. A list of the editing keys is available by

pressing the F1 key while the logging data form is displayed. The

fields and their sizes is as follows:

Field Name          Size           Starting position in log file

DATE                  6               1

TIME                  5               7

CALL                 11             12

NAME                 20             23

QTH                  39             43

MY RST                7             82

HIS RST               5             89

MODE                 10             94

28

COMMENTS            48             104

FREQUENCY            7             152

POWER                 4             159

QSL                   4             163

OTHER                12             167

ENDTIME               4             179

Which looks like about a 182 character record in the flatfile

mode. I hope all of these fields are self-explanatory. The OTHER

field is just an additional field that can be used to put things I

didn't think of.

If you elect not to use the flatfile mode your file will be in this

format

"103193","0532","k4gvo","Jim","Fayetteville, GA",

"599","599","","10.109","100","","","","0650"

Of course,  it will be all on one line.  This format is called

"comma delimited" by *base and others of its ilk.  This format

may be imported to many of the database managers and spread-

sheets available on DOS. If you don't like the layout, drop me a

note.

9     Selective calling feature

Here is part of a typical init file that will support the selective

calling feature.

!KEY_F10 togsel

RSELCAL k4gvo K4GVO W4LYBZW k4gvozw

ENDCAL ZCZC zczc NNNN nnnn

ALLCAL ALLSTAT

SELMAXT 240 seconds

EXTEND MXLEN 600

SELREPLY \rx\r^o^o^o^o^o^ode k4gvo %D ^m^d

If, in the body of a message being recorded, the string "MXLEN"

is found, the time limit will be bumped up to 10 minutes.  Note

that the "seconds" field on the SELMAXT option line is really a

comment and not a specification.  The following line WON'T set

the time to 2 minutes.

SELMAXT 2 minutes

10      License

I'm not going to bother with the usual nonsense about responsibil-

ity and guarantees. If you're foolish enough to use a free package

29

with something that you think is critical enough that you might

be tempted to sue someone if it screws up, you deserve what you

might get.  CAVEAT EMPTOR! I hope I spelled that right.  My

last Latin course was before my first FORTRAN II course.  Am I

telling my age?

I guess I didn't mention that this program is almost free. Might

even be freeware, but since freeware is a registered trademark, I

understand, I won't call it that. If you use this program and feel it

is of any value, please drop something in the next Salvation Army

kettle you see.  If that activity offends you for any reason, please

just do something nice for someone that you wouldn't normally do.

Like buy your secretary lunch the next time she screws up royally

instead of reaming her out.

You can use it, change it, abuse it, distribute it, charge for it

or most anything else you want to do with it, except if you charge

more than a reasonable copying charge for it, you must tell whoever

you sold it to that they could have gotten it for nothing if they had

just looked around. You may distribute it in any form, as long as

this file is included in its original form. You may add as many files

as you wish.

If you want source, drop me a note and we'll discuss it.  I'll

probably eventually turn it out into the public domain, but I'd be

too embarrassed to do so with it now.

11      In Case of Trouble

Problem: You hook up the TNC and it won't talk to you.

Solution:  Check the com port parameters.  If you are absolutely

sure that they are right, try turning the TNC off and then back on.

If you see the restart message on the screen, then you've probably

got the flow control set wrong or an open wire in your RS-232

cable. Set the flow control to none and restart SimpTerm. If other

terminal programs see to work, but SimpTerm doesn't, it is usually

the flow control.

If you don't see anything when you power-cycle the TNC, then

you still might have a flow control problem or a com port setting

problem. Be real sure that you have selected the right com port.

Problem: You hook up the TNC and it prints garbage or a foreign

language.

Solution: This is usually caused by either an incorrect parity setting

or an incorrect speed setting. If you see what look like words, but

30

half or so of the letters are wrong, it's probably the wrong parity.

If it is total garbage, change the baud rate.

12      Error messages

I hope they are few and perfectly understandable. I haven't com-

piled a complete list, but here are a few you may encounter during

the initialization phase.

HELP line has no file name

The HELP line must have a valid file name as the first parameter.

LOGFILE line has no file name

The LOGFILE line must have a valid file name as the first param-

eter.

PRELOAD has no file name

The PRELOAD line must have a valid file name as the first pa-

rameter.

MID has no value

The MID line must have a valid non-zero, non-blank value as the

first parameter.

Key table corrupt

This is an internal error, it should never appear unless memory

gets overwritten.

Line nn, Invalid key name <keyname>

SimpTerm can't identify the key name on the line, number nn.

Check your spelling.

Malloc failed

SimpTerm ran out of memory.

Number of lines, nnn exceeds max, mmm

The maximum number of continuation lines has been exceeded.

Port definition in error, line nnn

There is something wrong with the COM port definition on line

nnn. Be sure it looks like the examples given above.

31

Port init failed nn

The attempt to initialize the com port failed with an error of nn.

ERROR (some message)

Messages of this type will be returned from the CD-ROM database

access. If the message mentions MALLOC, then you probably ran

out of memory.  Try to use the CDSWAP parameter in your .ini

file.

If the message involves a file access error, open, read, etc. then

check your QRZ or AMSOFT parameters to be sure the drive/path

is correct.  If the message concerns the "list" processing then you

may have run out of memory.  In that case try the CDSWAP pa-

rameter.  This is also true if you get the "ERROR not loaded"

message.

If you receive an error message that isn't mentioned above it

may be an internal error and one that "shouldn't" ever happen,

but did.  In that case, please write me for assistance.  Describe

what you were doing at the time, and what brand of TNC you are

using. Anything else that you can thing of would be helpful too.

13      Final notes

I hope that you find this program easy and enjoyable to use.  I

wrote it while I was traveling with a T1000SE (8086) laptop (no

hard drive), a PK-232 and a IC-730.  I didn't really like to use a

simple terminal program and felt the big gun programs, just were

too clumsy for portable use.  I've made a few enhancements to

that original program and hope that you will find it acceptable. I

suspect that you aren't going to find a cheaper product.

This manual was typeset with LATEX. I will be offering a hard-

copy of this manual on 8.5 x 11, 3 hole punched paper ready for

your 3 ring binder in the near future at what amounts to repro-

duction and mailing costs. If you are interested please drop me a

note with a sase.

If you like SimpTerm. Tell others about it. If you don't like it,

please tell me why. If you don't like it and tell others that it isn't

any good, then we all lose. You lose, 'cause I just might be able to

make it what you wanted, if I only knew, others lose 'cause they

might find something in it for them that you weren't interested in

and I lose, 'cause my feelings have been hurt, so PLEASE if you

want to complain, fire it my direction. I can't promise I can make

it your personal rose garden, but I will try to fix any problems or

deficiencies that might show up.

32

The documentation is slowly improving, but not quite as good

as I would like to have it. I sure had fun with the spell checker. It

wanted to make rtty into ratty and amtor into actor. Wonder if it

knows something I don't?

14      Bugs

I think I have fixed all that have been reported.

I'm not happy with the editor in the macro function,  but I'm

rapidly running out of memory to make enhancements.  I don't

want to convert to a 32 bit format because I want to be able to

continue to support the old platforms.

The com configuration menu is only usable with a mouse.  There

isn't a keyboard interface.

15      Futures

Clean up the code so I'm not embarrassed to distribute it.

I'd like to add a more robust station logging function or maybe

multitask with the command processor so one could run their fa-

vorite logging program at the same time with out having to use

DV or something similar.

I'm working on a Linux version of SimpTerm as well as a Windows

version.  The Linux version is handicaped by the lack of a decent

toolkit. I don't want to use Motif 'cause they want money for it and

it doesn't make sense for me to give something away only to help

line the pockets of someone less generous.  The Windows version

was stuck for months trying to find a bug. I find Windows a very

hard environment to program in unless you work on it constantly.

There is so much to remember that if you are away from it for a

while you almost have to start again.

33

Index

active call, 8, 11, 27               copying, 30

ALLCAL, 22                           CTRL, 6

ALT, 6

ampersand, 9                         database, 29

AmSoft, 12, 27                       date and time display, 19

auto, 10, 13                         dates and times in strings, 7

autoexec.bat, 20                     day, 8

AUTOTIME, 13, 20                     DAYTIME, 18

DDMMYY, 24

backslash, 9                         DEBUG, 19

backslash character, 7               default, 16

backspace, 15                        DEL_EQ_BKSP, 18

backspace key, 18                    delete key, 18

base address, 26                     DesqView, 18

baud rate, 26                        dispbut, 10, 25

bauddown, 10, 12                     display, 5

baudup, 10, 12                       dos, 10

binary data, 11, 15                  dos, entering, 12

BINARYCAP, 11

BINARYLOAD, 15                       echo, 5

bios, 23                             end, 15

bugs, 33                             END key, 15

button, 24                           ENDCAL, 22

environment variable, TZ, 20

call letters, 12, 27                 Error messages, 31

calls, incoming, 11                  Europe, 24

capture, 11, 25                      exclamation, 15

CD, 12, 27                           EXTEND, 23

CD-ROM, 12, 19, 27

CD-ROM support, 27                   file, color configuration, 25

CDSWAP, 19                           files, assigning to function keys,

color configuration, 25                       9

color, changing, 4                   flatfile, 29

color, configuration, 25             FLATLOG, 23

COM port, 16, 26                     flow control, 17

command line options, 25             function key, 4, 15

comment lines, 16                    functions, assigning to keys,

comments, 16                                  10

confcom, 10, 15                      functions, list, 10

config, 10, 25

continuation, 9                      getcall, 10

control character, 9                 GMT, 20, 21

control characters, 7, 21            HELP, 17

34

help, 10                             quiet, 25

help function, 11, 17                quit, 10, 15

hour, 8

rback, 10, 12

initial file, 5                      rback1, 10, 12

initialization file, 4, 25, 26       receive, 5

irq, 26                              reset, 10

RESLCAL, 21

keyboard, 33                         return character, 7

Keyboard assignments, 5              rforward, 10, 12

keyboard macros, 8                   rforward1, 10, 12

keybrcl, 10, 14                      rlarger, 11, 12

rsmaller, 11, 12

License, 29

linefeed character, 7                savebuf, 11

log, 23, 33                          scroll bar, disabling, 19

log file, 24                         scroll bars, 27

logbook, 28                          scrollbar, 24

LOGFILE, 24                          scrolling, 12, 19

logit, 10, 11                        SCROLLLINES, 19

lookup, 12, 27                       selcal, 20

selective calling, 20, 29

macro, 10, 12, 13                    SELFILE, 20

menu, 4, 10, 12                      SELMAXT, 22

MID, 18, 25, 26                      SELREPLY, 21, 22

minute, 8                            SENDKEY, 14, 15

monochome, 25                        SHIFT, 6

month, 8                             simp.cfg, 25

mouse, 24, 33                        simp.ini, 23

Mouse usage, 11, 27, 28              size of transmit window, 26

multi-tasking, 18                    SNOW, 23

MYCALL, 4, 6, 8                      source, 30

NODV, 18                             special formatting characters,

NOMOUSE, 27                                   21

NOSCROLLBAR, 19                      startup screen, 24

notes, 32                            stationslog,t11,a23,t24,u28s, 5, 11

parity, 26                           status line, 5

PK-232, 21                           stop, 15

POSTLOAD, 19

pound sign, 16                       tab character, 7

PRELOAD, 18                          TIMELINE, 19

pull-down menu, 11, 28               tkeybuf,t11,o14gcap, 11

QRZ, 12, 27                          togsel, 11

Quick start, 4                       transmit, 5

35

Transmit window, 5

transparent data, 11, 15

trouble, 30

TZ, 20

upload, 11, 15

video, 23

window size, specifying, 18

wrap, 25

xback, 11, 12

xback1, 11, 12

xforward, 11, 12

xforward1, 11, 12

year, 8

YYMMDD, 24

36