WESTERN DIGITAL

DIAGNOSTIC PROGRAM FOR

"PLUS"-FAMILY LAN ADAPTER BOARDS

RELEASE INFORMATION

Version 2.3               19 May 1988

OPERATING CONSIDERATIONS

When using this version of the Diagnostic Program with the

EtherCard PLUS With 32K RAM And Boot ROM Socket (WD8003EBT

adapter board), only certain values should be selected in

the RAM Base Address field of Test 1 (Configuration).  The

values you should select are based on whether the W7/W8/W11

jumpers on the board specify 32 Kbytes or 8 Kbytes of RAM

buffer:

RAM Buffer Size       RAM Base Address Values

---------------       -----------------------

32 Kbytes          C0000, C8000, D0000, D8000

8 Kbytes           C0000, C2000, C8000, CA000,

D0000, D2000, D8000, DA000

VERSION HISTORY

-- Version 2.3 (Current Version) --

Addresses C8000 and CA000 were added to the choice of RAM

Base Address parameters in Test 1 (Configuration).

-- Version 2.2 (25 January 1988) --

Previous version.

##