IOMON - Protected mode I/O port monitor

---------------------------------------

IOMON is a protected mode I/O monitor written by Rick Knoblaugh and

published in an article in Programmer's Journal (volume 9, number 5).

The program files for IOMON are copyright 1991 ASMicro and distributed

here with their permission.

IORPT is a reporting program that reads the memory resident data buffer

maintained by IOMON and produces several reports. IORPT was written by

Terry Hughes of TurboPower Software and is released to the public

domain.

Version 1.0

1/92

Overview

--------

IOMON and IORPT can be used to monitor and report on the serial port I/O

performed by any communications program. As you might guess by their

names, IOMON does the monitoring and IORPT does the reporting.

This combination of programs can add some powerful and convenient

debugging capabilites to your toolbox. Previously, if you wanted to

monitor the exchange of data in a serial link you needed a data scope or

an extra PC (and cables) equipped with data scope-like software. While

there will still probably be times when only a data scope will do, IOMON

and IORPT make it possible to monitor many such communication exchanges

directly on one of the participating machines.

IOMON is a memory resident monitor that tracks all accesses to a

specified range of I/O ports. As presented here, its intended purpose is

to monitor the range of I/O ports that belong to a UART. The tremendous

utility of such a program is that it can monitor communications programs

as they run, without interfering with them, and note all serial data

input and output as well as modem and line control performed by that

program.

IOMON was originally published in Programmer's Journal as a general

purpose I/O port monitor. We've made one small modification to it to

allow the its information buffer to be truly circular (as published, the

buffer was cleared everytime it wrapped).

We've supplied the file IOMON.EXE executable file as well as the source

code files distributed with the Programmer's Journal article (with our

small modification). For more information about how IOMON works we

suggest you read Rick Knoblaugh's article on the program in Programmer's

Journal 9.5.

IORPT is a reporting program that reads IOMON's memory resident data

buffer and produces two reports -- a "trace" and "audit" report. The

trace report is identical to the Tracing facility provided by Async

Professional. The audit report is similar to Async Professional's event

logging report except that it lacks a time stamp.

Note that while IOMON is a general purpose I/O monitor, IORPT assumes

that IOMON is currently being used to monitor the UART ports of COM

port. Running IORPT while IOMON is monitoring some non-UART port

addresses will give unpredictable results.

Using IOMON and IORPT

----------------------

In a nutshell, a typical testing scenario will consist of

1) loading IOMON with desired UART addresses

2) running the communications program you wish to monitor

3) running IORPT to build a report of what the communications program did

Here's a bit more detail about each step.

IOMON.EXE is a TSR that takes two command line arguments -- the starting

and ending port addresses you wish to monitor. For example:

IOMON 3F8 3FE

will monitor all inputs and outputs to I/O ports $3F8 through $3FF. It

maintains an internal 8K buffer of information about all accesses to

those port addresses. For each access it saves the exact port address,

the data input or output and whether the access was a word or byte

access.

These addresses are assumed to be in hexadecimal notation and the first

address must be less than the second address. When using IORPT in

conjuction with IOMON, these addresses will be the UART port addresses

of the COM port you wish to monitor. Below are the standard addresses

for COM ports:

COM1  3F8 3FF

COM2  2F8 2FF

COM3  3E8 3EF

COM4  2E8 2EF

IOMON isn't restricted to just these addresses. It will attempt to

monitor whatever range you give it. However, IORPT assumes that you are

using it to monitor a UART. To report on other addresses, you'll need to

use the more simple-minded IODISP.EXE program that is part of the

original IOMON package.

To perform its monitoring, IOMON must run in virtual-86 protected mode.

This provides two obvious restrictions on its use: 1) it requires a 386,

486 or later, and 2) the machine cannot already be running in protected

mode. Since most memory managers (QEMM and 386Max) also place the

processor in protected mode, they must be removed before running IOMON.

An additional restriction of the way IOMON operates is that it cannot be

uninstalled by itself or by using external mark/release programs. Once

you are finished with your monitoring tests you will need to reboot your

machine to remove IOMON.

Assuming you've installed IOMON correctly, the next thing to do is run

the communications program you wish to monitor. Each time that program

accesses one of the ports in IOMON's range, an I/O exception will be

generated and IOMON will temporarily gain control. It will allow the I/O

to continue but will peek at the data that was input or output and add

an entry to its circular log. The circular log will contain the last

8192 I/O events.

When the communications program is finished you will use IORPT to read

that circular buffer and produce its reports. IORPT has the following

command line arguments:

Usage: IORPT [options]

/T      build standard APRO trace to IORPT.TRC

/D      build detailed audit report to IORPT.AUD

/F      filename to use (no extension)

The /T option builds a report identical to Async Professional's Tracing

report. Here's a short sample:

Transmit:

ATZ[13][10]

Receive:

ATZ[13][10][13][10]OK[13][10]

The program begin monitored sent an 'ATZ' to the serial port and

received back on echo of that command plus an 'OK' response.

The /D option builds a report similar to Async Professional's Event

Logging report. However, this report doesn't provide and timestamp and

is otherwise different enough that we gave it a different name -- an

audit report. Here's a short sample:

Addr  Direction  Value   Message

----  ---------  -----   -------

02F9     In        00    Reading interrupt enable mask

02FA     In        01    Reading interrupt type

02FC     In        00    Reading modem control register

02F9     In        00    Reading interrupt enable mask

02F9     Out       00    Setting interrupt enable mask

02F9     In        00    Reading interrupt enable mask

02F9     Out       00    Setting interrupt enable mask

02F9     In        00    Reading interrupt enable mask

02FC     In        00    Reading modem control register

02FB     In        03    Reading line control register

02FB     Out       83    Enabling DLAB

02F8     In        30    Reading baud rate

02F9     In        00    Reading interrupt enable mask

02FB     Out       03    Setting line control register (DLAB off)

02FF     In        F0    Reading scratch register

02FF     Out       F0    Writing scratch register

The information should be fairly self-explanatory. Each line represents

one access to a UART port. Addr has the exact address, Direction tells

whethe this was due to and IN or OUT instruction and Value contains the

value input or output. Message is a short text explanation that IORPT

attaches to each address, direction and value, assuming that the address

range being monitored is part of a UART address range.

Note that you can produce both reports by specifying both /T and /D on

the command line. When debugging information exchanges, like protocols,

you probably find the /T report helpful. When debugging low-level

problems (like modem control issues) you'll probably find the /D report

more helpful.