Synchronet Multinode Documentation

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Synchronet Version 1a runs under DOS v3.0 or greater and SHARE (or NetWare).

A DOS process is required for each node on a Synchronet system. You can run

multiple nodes on one cpu with DOS multitaskers such as DESQview, Windows,

or OS/2 v2.0 or you can run one node per CPU and let the CPU's share the same

data on a common hard disk via Local Area Network (LAN).

LAN Method

~~~~~~~~~~

Using one CPU per node is by far the fastest (and possilby cheapest) way to

run multiple Synchronet nodes. Since Synchronet runs in 8086 real mode, any

IBM PC can run Synchronet with sufficient speed. The minimum equipment

necessary for each node would be a power supply, motherboard, at least 500k

free memory, a network interface card (NIC), floppy drive and controller (or

just a boot PROM for the NIC), and a modem. Some BIOS's require a video

controller and/or a keyboard as well, but they are not necessary for the

execution of Synchronet.

To link these computers together, you will need some form of networking

software. DOS based networks (Lantastic, NetWare Lite, CBIS Network O/S,

MainLAN etc.) are the cheapest and don't require a dedicated CPU as a file

server, but they lack the speed of a dedicated file server and network

operating system. The most popular Network Operating System (NOS) is Novell

NetWare. The disk performance on a dedicated NetWare file server is often

preferable over a local hard disk (especially on XT's) due to NetWare's high

speed file system and dedicated memory for intelligent disk caching. Synchronet

will run on any NOS that allows multiple CPU's running DOS to access data on a

common disk.

An alternative to using an individual computer for each node would be to use a

"computer on a card" board. Currently the most popular and reliable company

that produces these is Cubix Corporation. You can purchase a 4 XT board that

plugs straight into a NetWare file server bus slot for under $1500. You will

need a normal workstation for monitoring the nodes and performing local

maintenance, but you could make that workstation a node as well. So you can

have excellent performance with up to 21 nodes on just two computers (the

workstation and the file server). And external expansion buses are available,

each holding up to 12 cards (48 nodes) without adding any more individual

computers. You will need an external modem and serial cable for each node.

Multitasker Method

~~~~~~~~~~~~~~~~~~

Any program that allows the simulataneous execution of multiple DOS programs

should be able to run multiple Synchronet nodes per CPU. To reliably and

efficiently run multiple DOS sessions on one CPU, you will probably need an

80386 CPU (the faster, the better), and at least 2 megabytes of memory

(the more, the better). Different multitaskers will have different hardware

requirements, but the common denominator seems to be an 80386 and extended

or expanded memory.

You will need a separate IRQ line and UART I/O address for each COM port in

the computer. If you wish Synchronet to always use BIOS calls for screen

output, you need to include the B parameter on the SBBS command line. Normally,

Synchronet uses direct video for block text which can conflict with some

multitaskers that manage a split screen for multiple DOS sessions.

Generally, the faster the machine, the more nodes you can run. But how many

for what kind of performance is not an easy question to answer. It depends

heavily on the multitasking software, modem speed, and hardware I/O. Under any

configuration, expect some amount of performance degradation when running

multiple nodes per CPU.

Is it Safe?

~~~~~~~~~~~

Many Sysops will worry how reliable the system is with multiple simultaneous

users. Rest assurred that Synchronet was designed from the ground up for

multiple simultaneous users. Data integretity and system fault tolerance is

never sacrificed when adding nodes to a Synchronet system.

Users can post on the same sub-board, e-mail the same user, download the same

file, run the same external program, perform virtually any system function

simultaneously. Note: External programs must support multiple simultaneous

users if the program has been configured for multiuser access from Synchronet.

Considerations and precautions have been taken where user data is concerned

to allow changes to the real-time database to take place immediately - even if

the user is online on a node other than the one that performed the data update.

Any security, configuration, or statistics changes made remotely will take

effect immediately.

The transfer section disallows simultaneous uploading of the same filename to

the same directory and prevents a file that is currently open (being downloaded

or just added to a user's batch download queue) from being moved, removed, or

edited while allowing simultaneous downloads.

As a general rule, no data files or records are allowed exact simultaneous

access if one of the nodes will modify the data. Simultaneous read only access

is allowed. Collisions (simultaneous write access requested of a file by two

or more nodes) are logged along with the retry count. There is a retry maximum

count (or time-out) that will deny access to the requesting node and log a

critical error for that node. This situation should not occur, but provisions

are made for it in case of hardware failure of a node with a file or record

open with write access.