���������������������� FrontDoor 2.20c setup for Vcom 0.2b �������������������

This document explains how to configure FrontDoor 2.20 (multinode) to run

under Vcom 0.2b, a Windows 95 inbound telnet driver that emulates a modem.

You may ask 'Why would anyone want to run their mailer on a telnet node?'

A few DOS based BBS Programs (Such as ProBoard and RemoteAccess) have

trouble waiting for a call when they are not the forground Windows task.

Often they do not see the RING from the modem untill the second or third

ring. This can cause problems for VCOM, which also sends RING and waits

for an ATA command. Using a front-end to answer and pass the caller to

the BBS solves this.

But imagine if Vcom also supported OUTBOUND telnet. Then if FD issued an

ATDT1-805-494-8327 for example, if Vcom's modem emulator could convert

that to the dial command to the desired IP number to telnet to, it could

connect with another telnet mailer, and transfer mail packets and do file

requests just as if it was done using modem lines!

03/25/97

------------------------------------------------------------------------------

How to install Vcom with FrontDoor.                                Mike Ehlert

This assumes that you already have FrontDoor 2.20c, Windows 95,

and your BBS software up and running.

1. Create a directory for VCom, unzip the archive there and read

the documentation.

2. Edit the system.ini file in your Windows directory, and find

the section called [386Enh]. Below that line, add the following:

device=c:\vcom\vcom.vdx

(using the proper path to the directory name you created.)

3. if you want to use a base address or IRQ other then the default

recomended one, then you will also need to add some additional

lines to your system.ini. (refer to the vcom docs about this).

VCOM defaults to using 2A0 as the base address, and IRQ7.

These should both work fine, even though your printer port is

probally already using IRQ7 they should not conflict with each

other. But if you have any other devices (such as a sound card)

using the same resources as VCom then there will be a conflict.

Check which IRQ your sound card is on. Usually IRQ5 is used for

the sound card, but occasionally they are configured for IRQ7.

You can look in the control panel to see what addresses and IRQs

belong to which devices.

(Right click My_Computer, click Propertys>Device_Manager>Computer.)

Once you are done editing your system.ini, you will need to reboot

the computer for the changes to take effect!

4. Create a new modem config for FrontDoor. Run FDSETUP.EXE and

select MODEM > TASK_SPECIFIC and select the task (node) number

for the telnet node. Under HARDWARE, set following:

Serial port                   4

Locked port                   Yes

Maximum baud rate             Fixed

Reset modem to connect speed  No

Lower DTR when busy           No

Lower DTR to terminate call   No

Toggle DTR before dialing     No

Carrier detect mask (CDMASK)  128

I use COM4 for VCom here, but you could use any number for the

serial port up to 32 (thats the highest that FD supports).

Be sure that you do NOT choose a serial port number that really

exits in your system.

Under CONNECT MESSAGES, set the following:

300         CONNECT|

1200        CONNECT 1200/

1275        CONNECT 1275

2400        CONNECT 2400

4800        CONNECT 4800

7200        CONNECT 7200

9600        CONNECT 9600

12000       CONNECT 12000

14400       CONNECT 14400

16800       CONNECT 16800

19200       CONNECT 19200

[snip]

The rest don't matter, and probally the only one that does

matter is the last one, as VCom reports "CONNECT 19200 /TELNET"

Under STATUS MESSAGES, it should look like this:

Error       ERROR

Busy        BUSY

No Carrier  NO CARRIER

Ok          OK

Ring        RING|

No Dialtone NO DIAL

No Answer   NO ANSWER

Voice       VOICE

Under ANSWER CONTROL, it should look like this:

Manual answer Yes

Force answer  ATA|

Answer delay  1

Enable LDFRS  No

5.  Edit your FrontDoor/BBS Batch File.

I'm assuming this batch file is where your fossil driver is being

loaded. VCom Server must be loaded before the fossil driver is

loaded, so that it can emulate the com port. VCom works with ADF,

BNU, and X00 Fossil Drivers. It will NOT work with WinFossil.

Here is a (simplified and documented) version of my Batch file.

Much was stript out, but the barebone version is fully functional.

echo off

rem   ���������������������������������������������������������������������Ŀ

rem   � FrontDoor / ProBoard BBS / FastEcho / VCom / Multinode Batch File   �

rem   �                                                                     �

rem   � usage: RUNFD.BAT [node number]                                      �

rem   �������������������������������������������by Mike Ehlert��1:102/1001��

rem

rem   NODE 1 : COM1 IRQ 4 locked at 115200  Modem

rem   NODE 2 : COM2 IRQ 3 locked at 115200  Modem

rem   NODE 3 : COM3 IRQ 5 locked at 115200  Modem

rem   NODE 4 : COM4 IRQ 7 locked at 115200  Telnet (Using VCom)

rem   NODE 5 : Local use only

rem Set differnt port environment variables, depending on the node number.

if "%1"=="" goto usetask

if %1==1    goto node1

if %1==2    goto node2

if %1==3    goto node3

if %1==4    goto node4

if %1==5    goto node5

echo bad node number passed, must be 1-5.

goto end

:USETASK

rem  no number was passed on the command line, so check if the %TASK%

rem  variable was already defined earlier..

if "%TASK%"=="" goto NOTASK

if %TASK%  ==0  goto NOTASK

rem if %TASK% is already defined, assume all variables are defined.

:NOTASK

echo you need to pass the node number on the command line.

goto end

rem Standard COM settings

rem =====================

rem      COM1 = 3F8,IRQ4

rem      COM2 = 2F8,IRQ3

rem      COM3 = 3E8,IRQ?

rem      COM4 = 2E8,IRQ?

rem =====================

:NODE1

set PORT=1

set BASE=3F8

set IRQ=4

goto setup

:NODE2

set PORT=2

set BASE=2F8

set IRQ=3

goto setup

:NODE3

set PORT=3

set BASE=3E8

set IRQ=5

goto setup

:NODE4

set PORT=4

set BASE=2A0

set IRQ=7

rem  The following line adds a FD semephore to tell FD to never dial out

rem  on node 4, since outbound telnet is not functional in VCom 0.2b.

rem>c:\fd\fdsem\fdnocall.4

goto setup

:NODE5

set PORT=0

set BASE=XXX

set IRQ=X

:SETUP

set TASK=%1

set NODE=%TASK%

set LOCK=115200

set DSZPORT=%BASE%,%IRQ%

set DSZLOG=DSZ.LOG

rem  ansi.com is faster/smaller then ansi.sys.

c:\tools\ansi.com

rem  Load VCom Server before loading the fossil driver.

rem  Only load it if the current node (task) is the telnet node.

if %TASK%==4 c:\vcom\vcom.exe

rem  Load the fossil driver (I use ADF).

c:\tools\adf.exe com%PORT% %BASE% %IRQ% %LOCK% 4096 4096 14

rem  FrontDoor has the same problem as many DOS applications, it does

rem  not release idle time properly when it's monitoring the COM port

rem  waiting for RING, when it is not the forground windows task.

rem  FDFIXW4W is a small TSR that makes FD think it's in Desqview.

rem  It is available at any FD support site.

c:\fd\fdfixw4w.com

:START

rem Unless this is a local node, run FrontDoor now.

rem First make sure there are not any old dobbs[node].bat files,

rem FD does not need to be passed the node number, as it automaticly

rem looks for the %TASK% environment variable, and uses that as the

rem node number.

c:

cd \bbs\node%TASK%

if %TASK% ==5 goto local

if exist dobbs%TASK%.bat del dobbs%TASK%.bat

\fd\fd.exe

rem Check The errorlevel the FrontDoor exited with and process accordingly.

if errorlevel 151 goto BADCODE

if errorlevel 150 goto CLEAN

if errorlevel 130 goto UNPACK

if errorlevel 120 goto FAX

if errorlevel 101 goto BBSCALL

if errorlevel 100 goto LOCAL

if errorlevel 99  goto START

if errorlevel 20  goto BADCODE

if errorlevel 9 goto fd_err_9

if errorlevel 8 goto fd_err_8

if errorlevel 7 goto fd_err_7

if errorlevel 6 goto fd_err_6

if errorlevel 5 goto fd_err_5

if errorlevel 4 goto fd_err_4

if errorlevel 3 goto fd_err_3

if errorlevel 2 goto fd_err_2

if errorlevel 1 goto fd_err_1

if errorlevel 0 goto exit

rem *************************************************************************

:BBSCALL

rem Run the BBS software, using the (enhanced) DOBBS.BAT method.

rem

rem FrontDoor 2.20 creates a one line DOBBS[node].BAT file, which looks like:

rem

rem      EXEBBS 28800 3 401 3 /ARQ N/A

rem      |      |     | |   |  |    |

rem      |      |     | |   |  |    DFRS (CallerID)

rem      |      |     | |   |  connection type

rem      |      |     | |   node number

rem      |      |     | time untill next event (in minutes)

rem      |      |     port number

rem      |      baud rate

rem      exebbs.bat

rem

rem If Caller-ID (DFRS) is activated, then the DOBBS.BAT will look like:

rem

rem  EXEBBS 28800 3 401 3 /ARQ _DATE_=0125_TIME_=1259_NMBR_=8054948327

rem         ----- - --- - ---- ------ ---------- ---------- ----------

rem         %1   %2 %3 %4 %5   %6     %7         %8         %9

rem                            ---------------------------------------

rem                            the caller-id sequence is broken by '='

rem                            which acts as a parameter seperator.

rem

rem  The original DOBBS.BAT method was to have the EXEBBS.BAT file load the

rem  BBS passing the parameters as %1 %2 %3 etc..

rem  I do it a differnt way, by only using the EXEBBS.BAT to set five

rem  parameters as environment variables, and then it returns to the main

rem  batch file (this one) which loads the bbs using those variables.

rem  This also allows me to put the callers phone number in %CALLID%.

rem

rem  Here is my enhanced EXEBBS.BAT file: (which should be in the DOS %PATH%)

rem

rem  -snip-

rem         set BAUD=%1

rem         set PORT=%2

rem         set TIME=%3

rem         set MODE=%5

rem         set CALLID=%9

rem  -snip-

rem

rem  Running the DOBBS[node].BAT will result in setting the above variables.

call dobbs%TASK%.bat

rem  Now all the above variables are in memory, so we can load the BBS.

rem  First go to the current nodes directory.

cd\bbs\node%TASK%

\pb\proboard.exe -b%BAUD% -n%TASK% -p%PORT% -t%TIME%

:LEVELCHECK

rem     Check the errorlevel that the BBS returned.

rem     if errorlevel 100 goto semaphore

rem     if errorlevel  99 goto escape

if errorlevel   6 goto badcode

if errorlevel   5 goto new_n&e

if errorlevel   4 goto new_echo

if errorlevel   3 goto new_net

if errorlevel   2 goto sysop_next

if errorlevel   1 goto modem_error

goto start

:LOCAL

rem  This runs the BBS in local mode.

c:\pb\proboard -l -n%TASK%

if %TASK%==5 goto end

goto levelcheck

rem *************************************************************************

rem various errorlevel handling routines:

:UNPACK

rem Got some stuff from the mailer. Unpack and Import it.

\bbs\fastecho\fastecho toss

rem check the errorlevel that fastecho returned

if errorlevel 12 goto N&E_RECIEVED

if errorlevel 11 goto ECHO_RECIEVED

if errorlevel 10 goto NET_RECIEVED

goto START

:N&E_RECIEVED

:NET_RECIEVED

rem netmail is imported using feutil

\bbs\fastecho\feutil import -incl

:ECHO_RECIEVED

\bbs\fastecho\fastecho toss

goto start

:NEW_ECHO

rem New echo mail entered by user. Export it.

:NEW_N&E

rem New net and echo mail entered by user. Export it.

:NEW_NET

rem New netmail entered by user. Export it.

\pb\pbjam

rem pbjam reads proboards ECHOTOSS.LOG and creates RA ECHOMAIL.JAM

rem allowing fasrecho to not need to scan every area for new mail

\bbs\fastecho\fastecho scan

goto start

:SYSOP_NEXT

rem Sysop requested to log on locally after last caller (beep first).

rem This is not supported by ProBoard yet.

echo ^G^G^G^G^G^G^

goto local

:MODEM_ERROR

echo BBS Died: Modem/fatal error or disk full?

goto end

:FAX

rem A FAX machine is Detected! Run BGFAX to receive the FAX

cd\fd\bgfax

bgfax.exe /FAST:4 c:\fd\fax f%TASK%! Q /DTE:%LOCK%

rem did it work?

if errorlevel 4 goto faxrcvd

goto start

rem the FAX was recieved, now run faxrecvd.exe to inform the sysop via a .MSG

:FAXRCVD

faxrcvd.exe -d -bc:\fd\fax -fc:\fd\fax

if errerlevel 1 goto start

rem Erase the fax incoming log, so that each message to the sysop only lists

rem the current fax.

erase c:\fd\fax\faxin.log

goto start

REM**************************************************************************

REM Nightly Maintance stuff (errorlevel 150)

:CLEAN

[snip]

goto start

rem *************************************************************************

rem Error Handling

:FD_ERR_9

echo -=- FD ERROR: (9) unknown Error!

goto end

:FD_ERR_8

echo -=- FD ERROR: (8) FrontDoor already loaded!

goto end

:FD_ERR_7

echo -=- FD ERROR: (7) FOSSIL not loaded or detected!

goto end

:FD_ERR_6

echo -=- FD ERROR: (6) FOSSIL driver Error!

goto end

:FD_ERR_5

echo -=- FD ERROR: (5) Unable to initialize modem!

goto end

:FD_ERR_4

echo -=- FD ERROR: (4) Modem not responding!

goto end

:FD_ERR_3

echo -=- FD ERROR: (3) Insufficient disk space!

goto end

:FD_ERR_2

echo -=- FD ERROR: (2) External Error: Possible missing file!

goto end

:FD_ERR_1

echo -=- FD ERROR: (1) Internal Error: oh no!

goto end

:BADCODE

echo *** Unknown error code ***

goto end

:EXIT

echo ***

echo *** Forced Exit from Front-Door (ALT-Q)  ***

:END

echo *** System Down! Line %TASK%. ***

rem -------------------------------------------------------------------------

rem <end of batch file>

6.     Test VCom using an ANSI compatible telnet terminal. I use NetTerm,

but Commnet will also work. Both are Windows / Win95 applications.

You should make sure that all your doors work, as the doors which

do not communicate with the fossil driver will most likley not work.

I've found that a few doors run very, very slow under VCom, but most

work just fine. Also check any external protocols such as DSZ. I

use PDZmodem which is a freeware Zmodem 8k fossil driven with an

optional chat interface. You can find it the BBS Archives, at

ftp://pcmicro.com/file_transfer_protocols/

Don't forget to reboot or restart windows every day untill you

get VCom registered, as it only works for 40 calls and then to

use it more windows needs to be reloaded. :-(

The most annoying thing about this beta version of VCom is that

it will not hang up on the user when it sees ATH0 or a low DTR.

This means it up to the user to log off, unless you happen to

manually disconect him by clicking on 'disable server' followed

by clicking in 'enable server' again.

Please keep in mind that the current beta version of Vcom does not

support outbound telnet yet, so it is not possible to 'dial-out' to

another telnet mailer using this version of VCom.

If you don't have Vcom yet, visit www.netime.com/~infrared/vcom

-----------------------------

If you have more questions about setting up VCom with FrontDoor,

you can contact me at mike@pcmicro.com, or www.pcmicro.com.

<end of doc>