Basic States table for each of the protocols. There is a Rosetta Stone at

the bottom of the document.

I can't claim copyright for these tables, but they do work.

Mark Dignam - Perth Omen BBS - 3:690/660.0@Fidonet

-------------------------------------------------------------------------------

Xmodem CheckSum

Sender                                       Receiver

NAK

NAK

SOH 01 FE data[128] sum

ACK

SOH 02 FD data[128] sum

NAK

SOH 02 FD data[128] sum

ACK

SOH 03 FC data[128] sum

ACK

SOH 04 FB data[100] 1A[28] sum

ACK

EOT

ACK

-------------------------------------------------------------------------------

Xmodem CRC

Sender                                     Receiver

C

C

SOH 01 FE data[128] crchi crclo

ACK

SOH 02 FD data[128] crchi crclo

NAK

SOH 02 FD data[128] crchi crclo

ACK

SOH 03 FC data[128] crchi crclo

ACK

SOH 04 FB data[100] 1A[28] crchi crclo

ACK

EOT

ACK

-------------------------------------------------------------------------------

Ymodem Batch (usually CRC!)

Sender                                     Receiver

C

C

SOH 00 FF Y/Zmodem Header[128] crchi crclo

ACK

C

STX 01 FE data[1024] crchi crclo

ACK

STX 02 FD data[1024] crchi crclo

NAK

STX 02 FD data[1024] crchi crclo

ACK

STX 03 FC data[1024] crchi crclo

ACK

STX 04 FB data[1024] crchi crclo

ACK

EOT

ACK

C

SOH 00 FF NUL[128]

-----------------------------------------------------------------------------

Ymodem-G Batch (usually CRC!)  Sender just looks for NAK. Single Nak bombs

Transfer.

Sender                                     Receiver

G

G

SOH 00 FF Y/Zmodem Header[128] crchi crclo

G

STX 01 FE data[1024] crchi crclo

STX 02 FD data[1024] crchi crclo

STX 03 FC data[1024] crchi crclo

STX 04 FB data[1024] crchi crclo

EOT

ACK

G

SOH 00 FF NUL[128]

ACK

-------------------------------------------------------------------------------

Sealink - Don't wait for the ACK's, if they start arriving, just make sure

they stay within 6 blocks of blocks being sent.

Sender                                         Receiver

C

C

SOH 00 FF SealinkHeader[128] crchi crclo

ACK 00 FF

SOH 01 FE data[128] crchi crclo

ACK 01 FE

SOH 02 FD data[128] crchi crclo

NAK 02 FD

SOH 02 FD data[128] crchi crclo

ACK 02 FD

SOH 03 FC data[128] crchi crclo

ACK 03 FC

SOH 04 FB data[100] 1A[28] crchi crclo

ACK 04 FB

EOT

ACK 05 FA

C

SOH 00 FF NUL[128] crchi crclo

ACK 00 FF

-------------------------------------------------------------------------------

Zmodem - for futher on this - find Chuck Forsbergs document on the Zmodem

protocol. Its a bit much to go into here, but basicly the ^X char,

which is called a ZDLE, is the whole key. To send a ^X as part of

data, you must send a $18,$58, etc.

The various 'frames' (the ZRINIT, ZDATA etc) start with a ZDLE and

are fully variable depending on the exact frame type, the method of

Crc (CRC16 or CRC32) and the flags which are exchanged between each

system at start of transmission

Sender                                         Receiver

ZRQINIT XON

ZRINIT

ZFILE

ZRPOS

ZDATA

data[1024] crchi crclo

data[1024] crchi crclo

data[1024] crchi crclo

ZEOF

ZRINIT

ZFIN

ZFIN

OO

-------------------------------------------------------------------------------

Yapp - This is a specialised protocol used (at this stage) only on Packet

Radio. It requires a error link end to end, but compared to say

YmodemG or Zmodem, is a bit of a slug!

Lenblock is sent as a byte, and therefore Max Blocksize is 256.

Sender                                         Receiver

ENQ SOH

ACK SOH

SOH lenblock filename NUL size_in_ascii nul

ACK STX

STX lenblock data[lenblock]

STX lenblock data[lenblock]

STX lenblock data[lenblock]  {until eof}

BRK SOH

ACK BRK

If more files - goto SOH lenblock etc, else

EOT SOH

ACK EOT

-------------------------------------------------------------------------------

Ahh The Rosetta Stone.

SOH       = $01

STX       = $02

BRK       = $03

EOT       = $04

ENQ       = $05

ACK       = $06

NAK       = $15

CAN       = $18

CRCNAK    = $43

YGNak     = $47

ZDLE      = $18

Ymodem/Zmodem header block. Ymodem pads it out to 128 bytes with nulls, Zmodem

just sends it as is.

record =

filename : array[1..max] of char; { must be in lower case! Terminated }

{ with a null, and remove paths if  }

{ you like, but you don't have to.  }

{ if path left in, must use '/', ala}

{ unix, not '\' ala dos             }

separator : byte = $00            { End of filename                   }

length    : array[1..max] of char { this is optional, and is sent in  }

{ ASCII, using decimal digits from  }

{ $30..$39                          }

Separator : byte = $20            { separates the length and the time }

datetime  : array[1..11] of char  { Date time of the file in unix     }

{ style, ie number of seconds since }

{ 00:00 1/1/1970. Is sent in OCTAL  }

{ ASCII, using digits from $30..$37 }

separator : byte = $00            { End of datetime                   }

end;

Sealink is a little bit different - but send similar info.

Record =

Length    : Longint              { length of file in standard Intel   }

{ Hi_lo format                       }

TimeDate  : Longint              { TimeStamp of file, in standard     }

{ MSDOS format                       }

Filename  : array[0..16] of char { filename , terminated with a null  }

ProgName  : array[0..14] of char { sending programs name.             }

Ackless   : Boolean              { Can handle overdrive.              }

Fill      : array[0..86] of char { Nulls                              }

end;

-------------------------------------------------------------------------------